If you have suggestions for improvements or find any bugs please submit

via fax or mail.

EMAC Inc.

POB 2042

Carbondale IL, 62902

fax. 618-457-0110

This document supercedes ATBMON.DOC and documents the improvements made to

the monitor in version X2.1a.

The backspace key can now be used to erase a character.

Access to all of the 80535's SFRs is possible now through the addition of

the following: ADCON, ADDAT, CCEN, CCH1, CCH2, CCH3, CCL1, CCL2, CCL3, CRCH,

CRCL, DAPR, IEN1, IP1, IRCON, P2, P4, P5, PCON, SBUF, T2CON, TH0, TH1, TH2,

TL0, TL1, TL2.

The IE and IP SFRs were replaced with IEN0 and IP0 respectively.

DBYTE was modified to allow access to internal RAM addresses 80 to FF.

The HELP command was added which displays an abbreviated command summary.

UPLOAD/DOWNLOAD have been changed so all I/O is through the main serial port.

UPLOAD no longer gives the "LOADING" message and no CTRL-Z or RUBOUT are

output at the end of the hex file.  DOWNLOAD no longer expects or sends

a comment.  Also, it now grabs the checksum on the end record (though

it doesn't check it).

SEND has been eliminated.

Interrupt vectors for the 80535 have been added and are offset to 8000h.

8003H		Remap address for ext. interrupt 0.

800BH		Timer 0 interrupt.

8013H		External interrupt 1.

801BH		Timer 1 interrupt.

8023H		Serial port interrupt.

802BH		TF2 + EXF2 interrupt

8043H		IADC interrupt.

804BH		IEX2 interrupt.

8053H		IEX3 interrupt.

805BH		IEX4 interrupt.

8063H		IEX5 interrupt.

806BH		IEX6 interrupt.

GETTING STARTED

The following jumper settings are required on Rev 2 boards:

JP2	o o-o a		JP1	1 2 3 4 5 6

o o-o b			o o-o o o-o

1 3 5

To start the monitor after power up, you must connect the main serial (CN1)

port to a terminal or terminal emulator with no parity, 8 data and 1 stop

bit and a baud rate of 1200, 2400, 4800 or 9600.  Pressing a space key twice

will set the baud rate to that of the terminal and the logon message will

appear.  In this version, if any other key is pressed or if there is a

non-standard baud rate, the monitor will default to 9600 baud.

Once communication has been established you can now type "HELP" and enter,

to see an abbreviated command list.  Type DOWNLOAD and enter to ready

the monitor to receive an Intel hex file.  Send the hex file that is on

the distribution disk (the file with the ".HEX" extension) to the Micropac

535 using the text transfer (or file upload) function of your terminal

emulator.  Once initiated, you will see each line of the hex file as it

is loading into the board.

When it is done, you will see the "*" prompt.  You may now start the program

by typing "MGO 8000" and enter.

If you want to single-step or set breakpoints, don't execute the

init section of the program since this section changes parameters

used by the monitor.  Instead, start at the section you want to debug.

You may want to install a JMP instruction over the init section until

your program is debugged.