MX-11: A Multitasking Executive for the MC68HC11.

Version 2.0

Kevin J White

Copyright 1994 Kevin J White

3/14/94

Introduction

MX-11 provides a preemptive multitasking environment for the

Motorola MC68HC11 that can be used with programs written using

Dunfield Development Systems' MICRO-C/11 C compiler.

I created MX-11 to simplify the task of writing software for data

acquisition and house automation projects.  I wanted to be able

to write modular software in a high-level language that could

retrieve data from sensors or switches, operate on the data and

then feed the data to displays or control other peripherals.

MX-11 does this by providing a library of C callable functions

that allow multiple operations to be performed on one processor

in an event-driven manner.  Actions can be triggered by external

events or internally by timers or another software task.

I have used the software mainly on MC68HC11A8's using the BUFFALO

monitor, it can be configured for other monitors, and other

variants of the 68HC11 by modifying the equate files.

Using MX-11

MX-11 is used by defining one or more tasks in your code and then

activating them according to your needs.

A task is created by writing a C function, and then defining it

as a MX-11 task using the function deftask().  This task can then

be activated by passing a message from another task or at

specific time intervals by adding the task to the polling list.

When a task is running it can communicate with other tasks by

messages or semaphores.  It can also use sleep() to inhibit the

task for a specified period.

Page - 2

Tasks have a fixed priority that is set when the task is defined,

tasks have increasing priority as in the order they are defined.

Currently it is not possible to modify the priority once the task

is defined.  Whenever a task is made runnable or a running task

is blocked, MX-11 will enable the highest priority runnable task.

A suspended task will be reactivated when the task is unblocked

by, for example a timer expiring or a semaphore being set.

Compiling your program with MX-11.

In order to use the MX-11 functions they must be made available

to the MICRO-C source linker.  This is done by putting the

library files in the appropriate directory (usually \mc\lib11),

modifying the index file so that MICRO-C knows in which file the

functions are placed and then compiling and assembling the source

in the normal fashion to produce a hex file for download to the

target processor.  The header file "MX.H" must be included in

your program and mxinit() called before you use any of the MX-11

functions.

MX-11 takes about 1.5-2.5k of memory space (depending upon which

functions you link in).  There are some areas where it could be

shrunk a bit.  For most of the applications I have had so far it

has not been a critical problem.

Running your Program.

The hexfile should be transferred to the target using a terminal

emulation program and a monitor such as BUFFALO or MON11.  When

the program is started the mxinit() function will set up the data

structures for MX-11 and OC1 for its real-time clock.  The MX-11

function are then available for use.

Installation.

MX-11 should be installed on a machine that has the Dunfield

Systems Micro-C MC68HC11 compiler already in place.  I have used

it with both version 2.14 and 3.0 although the examples use

structs which are not provided in version 2.14.

1:   Decompress the archive using PKUNZIP with the -d option.

Copy all of the files from the "lib11" directory into your

LIB11 directory (usually c:\mc\lib11).

2:   Merge the file "MXINDEX.LIB" into the file "EXTINDEX.LIB"

which is usually in \MC\LIB11.  (This is so SLINK can find

the MX functions).  If you are using MON11 rather than the

BUFFALO monitor you should edit the MXVECT.EQU file.

Page - 3

3:   Edit and compile your Micro-C programs in the normal fashion

remembering to include the "MX.H" header file.  The MX-11

code will be linked with your software by the MICRO-C source

linker.

The example programs show the basic methods of using MX-11.

These programs are compiled to run on the hardware descried

below, they will need recompiling if your hardware configuration

is different.

Hardware Configuration

My designs have all used a MC68HC11A8 with the BUFFALO monitor

(version 2.5 in ROM or 3.4 in non-volatile RAM).  For the

development of MX-11 I used a system with 32k of SRAM from $0-

$7FFF and 32k of non-volatile RAM from $8000-$FFFF (this is a 32k

RAM in a DALLAS Smartsocket).  I use BUFFALO version 3.4 with the

registers allocated at $1000.  Programs (Including the example

programs) start at $1040.

MX-11 uses output compare 1 for its real-time clock to give the

main system tick at 20Hz.  It assumes an 8MHz clock.  If the

clock is different you will need to adjust the divide ratio

defined by TM1CNT in MXEQU.ASM  (The counter actually runs at

40Hz and divides by 2 because the maximum time for the counter is

32 milliseconds).

The SCI and its interrupt vector is also used by the serial port

driver.  No other hardware specific resources are used.  Note

that BUFFALO uses 0C5 and the XIRQ for its trace function.  Be

careful if you re using BUFFALO 2.5 as it doesn't use the XIRQ

and if you have OC5 connected to XIRQ as needed for version 3.4

then the system will crash when MX-11 initializes.

Interrupt Service Routines

Version 2.0 of MX-11 adds the capability of writing ISR's in C.

The interrupt service routine is written as a normal C function

that is entered as a result of the interrupt.  It is defined as a

MX-11 task using deftask().  The task is then bound to a

particular interrupt using defintr().  The task should run to

completion every time it is called.  Interrupts are disabled when

it is entered but they may be enabled if desired.  If this is

done and another interrupt occurs the interrupt nesting count

will be incremented.  This ensures that all nested interrupts are

processed before returning to the process level task that was

originally interrupted.

Page - 4

The ISR is passed the interrupt vector number as a parameter.  It

is thus possible to have a common interrupt routine for multiple

interrupt sources.  Be careful about allowing reentrant ISR's.

If you enable interrupts it is possible for the same interrupt to

occur again, while the first invocation is still executing.  This

is not allowed because the two invocations will be executing with

the same execution context: the second invocation will destroy

variables from the first.

It is also not permissible to allow the ISR to block.  Normally

the ISR should be as short as possible and communicate with

process level tasks by setting semaphores, sending messages or

using global variables.

The addresses of the interrupt vectors are defined in the

MXVECT.ASM file.  Edit this if your system uses has the interrupt

vectors at a different location.  The MXINT.ASM interrupt support

functions need to know the size of each redirected vector and the

address of the first interrupt vector, again edit MXVECT.ASM as

needed for your system.

On an 8MHz cpu it will take about 70 microseconds for the code to

start executing assuming the processor had interrupts enabled.

Example 3 measures the interrupt latency.  Much of the kernel is

interruptable to allow this.

ROM code

MX-11 can execute out of ROM.  The only function that would need

modification is the MXSTKCHK() function that patches the core

task dispatcher.  The stack check function is mainly for debug

purposes so this should not be a problem.

How does MX-11 work?

The core parts of MX-11 are in the file MXKERNAL.ASM.  The

central routine is MXSWCH: the task switcher.  Whenever a

significant event, such as the expiration of a timer, or any of

the MX-11 function calls are made, they exit through MXSWCH.  It

then searches through all the tasks that have been defined and

starts up the highest priority one that is runnable.

Information about the task is contained in the Task Control Block

(TCB) that is allocated for every task when it is defined.  The

address of the TCB is returned by deftask().  It should be saved

in a C variable as many of the MX-11 functions require that you

supply the TCB of the target task.

Page - 5

When mxswch is called, the current task has it's context stored

on the stack as if it had been interrupted.  MX-11 can then use

an RTI to resume a task.  It also saves the contents of the ?PARM

and ?TEMP locations that are used by the MICRO-C library.

The current status of a task is maintained in the "tstat" record

of the TCB.  A task can be dormant, sleeping, waiting on a

semaphore etc.  (The various states are defined in MXEQU.ASM).

Some of the function calls modify the state of a task.  For

example the sleep() call puts the task into a sleep state and

also sets up a timer to reawaken the task after a specified

interval.

MX-11 sets up OC1 as a real time clock to interrupt every 25

milliseconds, on alternate ticks the timer functions are

processed.  The timers use another important data structure, the

Event Control Block (ECB).  A pool of these structures is created

in the heap at initialization time and they are used for

messages, semaphores, event flags and the pollqueue.

One use of an ECB is for defining the pollqueue:  A task can be

linked onto one of five pollqueues.  Every task is sent a defined

message at the interval for that queue.  Currently there are

queues defined for 1/20 second, second, minute hour and day

polling.

Messages and semaphores also use ECB's, they are linked in a

chain to the destination task or semaphore.  When the message is

received or the semaphore is signaled the ECB is returned to the

pool.

Hints for writing software using MX-11.

Writing code for use in a multitasking environment requires

additional care over writing normal code, you must be prepared at

any moment for a task to be interrupted and be suspended, while

another task takes over the CPU.  You must ensure that whenever

this could happen all global data structures are in a coherent

state; for example if you had a structure that had an X and Y

coordinate values for writing to the screen, if one task had

modified X but had not yet modified Y and another task started

executing and used this intermediate value, it would update a

screen position that was neither the old position or the new one,

but a combination of both.

A typical program design would have each task waiting as an

infinite loop waiting for an event or message, it would then have

a switch statement and perform appropriate processing depending

upon the message.  After completion it would resume waiting for a

message.

Page - 6

You should use semaphores and messages to communicate between

tasks, and be aware of the activity of all tasks. If multiple

tasks need access to a common resource then you should use a

semaphore to control access and ensure mutual exclusion.

Cputstr() and Cprintf() use this technique to ensure that one

task does not interrupt data coming from another task.  You can

also disable interrupts, but this prevents any other tasks from

executing and is not recommended except for very short duration

sequences.

You should not spin wait on an event.  If you do, then lower

priority tasks will not be able to execute.  It is permissible

for main() to do this as there are no lower priority tasks that

have to execute.  Although since the nulltask() may never execute

it will not be possible to get the CPU utilization measurement.

Changes from Version 1.0

Version 2.0 has the following changes from version 1.0

1)   Removed the sendmsgd() call.

2)   Added capability of doing ISRs in C.

3)   Rationalized the use of timeouts on semaphores and messages.

There is only one call for each of these functions that now

includes timeout parameters.  These may be set to NULL if

timeouts are not required.

4)   Much of the kernel is now interruptable to improve interrupt

latency.  (Typically about 70 microseconds)

5)   Added event flags.  Similar to semaphores but multiple tasks

may wait on them and be resumed simultaneously.

6)   Added wait for character with timeout function (Cgetcr())

and one for sending characters without translation Cputcr().

7)   The communication routines now have there own predefined

semaphores, they do not need to come from the pool of ECBs.

The remainder of the document is a description of the MX-11

function calls.  Also study the example code to see how to use

them.

Please contact me if you find bugs or if there are any features

that you would find useful.  I would also like to know if you

successfully use the software.  Good luck.

Page - 7

Kevin J. White.

337 Pennsylvania Avenue

Los Gatos

CA 95030

Internet address kevin@novell.com

Page - 8

Summary of MX-11 functions.

---------------------------

Initialization functions

------------------------

defpoll(taskh,pollq,msg)           add task to poll list.

deftask(&task, stack_size, name)   initialize task.

defintr(taskh, vector_number)      bind task to interrupt vector

mxinit(stack_size,num_tcb,num_mcb) initialize MX-11.

Timer functions

---------------

gettime()                     get hardware timer.

getTOD(time)                  get time of day.

setTOD(time)                  set time of day.

sleep(timeout)                pause task.

Message Functions

-----------------

chkmsg()                      check if message available.

flushmsg()                    flush all messages for task.

getmsg(timeout)               get next message from queue.

sendmsg(taskh,msg)            send message to task.

Semaphore functions

-------------------

sclose(&sema)                 close semaphore.

sopen(initial_value)          get handle to semaphore.

ssignal(&sema)                signal semaphore.

swait(&sema,timeout)          wait on semaphore with timeout

Event Flag Functions

--------------------

efopen(initial_value)

efclose(&eflag)

efwait(&eflag, mask, timeout)

efset(&eflag, value)

efclear(&eflag, value)

Serial port functions

---------------------

Cgetc()                       get character from serial port.

Cgetcr(timeout)               get raw character with timeout

Copen(inbufsize,outbufsize)   initialize serial I/O.

Cprintf(format_string,args..) printf to serial port.

Cputc(c)                      print character to serial port.

Cputcr()                      printf raw character.

Cputstr(string)               print string to serial port.

Ctestc()                      test for character at serial port.

Page - 9

CGETC

PROTOTYPE:

char Cgetc ()

ARGUMENTS:

none.

RETURN VALUE:

character from serial port.

DESCRIPTION:

The "Cgetc" function obtains a character from the serial port.

Copen must have been called to initialize the serial port.

If there is already a character in the input buffer, the function

will return immediately.  If not the task will block waiting for

one.  Multiple tasks can wait simultaneously, although it will be

ambiguous which task receives which character.

EXAMPLE:

c=Cgetc(); /* Get a character from the serial port */

Page - 10

CGETCR

PROTOTYPE:

int Cgetcr (timeout)

ARGUMENTS:

timeout value in ticks

RETURN VALUE:

0-255          received character from serial port.

-1             no character received before timeout.

DESCRIPTION:

The "Cgetcr" function obtains a character from the serial port.

No translation of the character is performed.  Copen must have

been called to initialize the serial port.  If no character is

received before the timeout is reached, the value -1 is returned.

If there is already a character in the input buffer, the function

will return immediately.  If not the task will block waiting for

one.  Multiple tasks can wait simultaneously, although it will be

ambiguous which task receives which character.

EXAMPLE:

i=Cgetcr(20); /* Wait for up to 1 second to get a character */

Page - 11

CHKMSG

PROTOTYPE:

int chkmsg()

ARGUMENTS:

None

RETURN VALUE:

Zero if none available.

One if at least one message available.

DESCRIPTION:

This function checks for availability of a message without

blocking the task.

EXAMPLE:

if (chkmsg()) printf("Message is %d\n", getmsg());

else printf("No Message Available\n");

Page - 12

COPEN

PROTOTYPE:

Copen(int in_buffer_size, int out_buffer_size)

ARGUMENTS:

in_buffer_size      Size of input buffer to allocate (bytes).

out_buffer_size     Size of output buffer to allocate (bytes).

RETURN VALUE:

0 for success

-1 for failure.

DESCRIPTION:

The "Copen" initializes the interrupt driven serial I/O driver.

It allocates buffer space from the heap for circular input and

output buffers.  It is assumed that the monitor program will have

initialized the baud rate, and this is not changed (BUFFALO sets

it to 9600bps).

This function must be called before any of the communication

functions are called.  The driver is interrupt driven and uses

semaphores to block tasks waiting for data transmission or

reception.  The serial port routines use semaphores for mutual

exclusion of multiple tasks using the serial port.  Ensure that

there are three MCB's available when Copen() s called.

EXAMPLE:

Error=Copen(8,80);/* byte input buffer, 80 byte output buffer */

Page - 13

CPRINTF

PROTOTYPE:

Cprintf(char *format_string, arg, arg,..)

ARGUMENTS:

Same as standard printf()

RETURN VALUE:

None

DESCRIPTION:

The "Cprintf" function prints to the serial port as in the

conventional printf.  A semaphore is used to ensure that the

entire string will be printed as one stream, not being

interspersed with characters from other tasks.

If the serial transmit buffer is full or another task has the use

of the port, then the task will be blocked.

Cprintf() needs to be passed the number of parameters in the D

register, to achieve this it must be declared as a register

function.  This can be done by including mx.h in your program.

EXAMPLE:

Cprintf("This is some text to printf and a number %d\n",j);

Page - 14

CPUTC

PROTOTYPE:

Cputc (char c)

ARGUMENTS:

c              character to print.

RETURN VALUE:

none

DESCRIPTION:

The "Cputc" function prints one character to the serial port.

Copen must have been called to initialize the serial port.

If the transmit buffer has space the character is put into the

buffer and the function returns.  If the buffer is full, the task

is set waiting on a semaphore and will be blocked.  If multiple

tasks attempt to transmit simultaneously, the characters will be

transmitted in the order of function calls, regardless of the

priority of the calling task.  If the buffer is full there may be

multiple tasks waiting on the semaphore.

EXAMPLE:

Cputc('A'); /* Output the letter 'A' to the serial port */

Page - 15

CPUTCR

PROTOTYPE:

Cputc (char c)

ARGUMENTS:

c              character to print.

RETURN VALUE:

none

DESCRIPTION:

The "Cputc" function prints one character to the serial port

without translation of carriage returns.  Copen must have been

called to initialize the serial port.

If the transmit buffer has space the character is put into the

buffer and the function returns.  If the buffer is full, the task

is set waiting on a semaphore and will be blocked.  If multiple

tasks attempt to transmit simultaneously, the characters will be

transmitted in the order of function calls, regardless of the

priority of the calling task.  If the buffer is full there may be

multiple tasks waiting on the semaphore.

EXAMPLE:

Cputcr('A'); /* Output the letter 'A' to the serial port */

Page - 16

CPUTSTR

PROTOTYPE:

Cputstr(char *s)

ARGUMENTS:

char           pointer to a string

RETURN VALUE:

none

DESCRIPTION:

The "Cputstr" function prints a string to the serial port.  A

semaphore is used to ensure that the entire string will be

printed as one stream, not being interspersed with characters

from other tasks.

If the serial transmit buffer is full or another task has the use

of the port, then the task will be blocked.

EXAMPLE:

Cputstr("This is some text to print\n");

Page - 17

CTESTC

PROTOTYPE:

int  Ctestc()

ARGUMENTS:

None

RETURN VALUE:

0-255          Character from serial port.

-1             No character available.

DESCRIPTION:

Ctestc() tests for a character at the serial port.  If none

available then -1 is returned.  If a character is available its

value is returned.  This function does not block if no character

is available.

EXAMPLE:

if ((i = ctestc())==-1) printf("No character available\n");

Page - 18

DEFINTR

PROTOTYPE:

defintr( char *taskh, int vector_number)

ARGUMENTS:

taskh          task handle.

vector_number  interrupt vector for task. (Defined in MX.H)

RETURN VALUE:

none

DESCRIPTION:

The "defintr" function binds a task to an interrupt vector.  When

the interrupt occurs a new C context is set up and the function

called.  The function should exit in the normal fashion.  The

interrupted task will then be resumed.  It is not permissible to

block while in an ISR.  in general ssignal(), efset() or

sendmsg() should be used to synchronize process level tasks.

MX.H defines the interrupts that a task can be attached to

EXAMPLE:

char task1h;/* Handle for task1 */

task1()

{

...

}

task1h = deftask(&task1, 128, "Task One");

defintr(task1h, OC2); /* Attach to Output Compare 2 interrupt */

Page - 19

DEFPOLL

PROTOTYPE:

defpoll( char *taskh, int pollq, int msg)

ARGUMENTS:

taskh          task handle.

pollq          queue to attach task to.

msg            integer message to send to task at each wake up.

RETURN VALUE:

Zero if error, one if no error.

DESCRIPTION:

The "defpoll" function attaches a task to a poll queue to be sent

a message at regular intervals.  If the task is dormant it is

awoken with the message available as its argument.  If it is

awake the message is queued for the task to retrieve it by a

getmsg() call or the task will be re-awoken when the task becomes

dormant.  An error will be returned if an MCB cannot be obtained.

The messages are synchronous with the TimeofDay so for example

the hour polling will occur at the beginning of the hour.

The implemented polling queues currently are:

0    Once per tick (1/20 second)

1    Once per second

2    Once per minute

3    Once per hour

4    Once per day at midnight.

EXAMPLE:

char task1h;/* Handle for task1 */

task1()

{

...

}

task1h = deftask(&task1, 128, "Task One");

defpoll(task1h, 1, 255); /* Send value of 255 once per second */

Page - 20

DEFTASK

PROTOTYPE:

char *deftask( char *function, int stack_size, char *name)

ARGUMENTS:

function       address of function to be defined as a task

stack_size     number of bytes on stack to reserve for task

name           pointer to name of task

RETURN VALUE:

Pointer to Task Control Block for task().

DESCRIPTION:

The "deftask" function allocates a Task Control Block for the

task and initializes it with starting address and stack.  The

task is set into a dormant state.  Tasks are assigned priority in

ascending order of their definition so define low priority tasks

first.  A task is started by sending a message to it or putting

it on a poll queue.

The address of the TCB for the task is returned by deftask.  The

value should be stored in a variable for use as a task handle in

later MX-11 calls e.g.. to send a message to it.

EXAMPLE:

char *task1h; /* Handle for task1 */

task1()

{

}

task1h = deftask(&task1, 128, "Task One");

/* Provide for 128 bytes of stack and give it a name */

sendmsg(task1h,0); /* Start it */

Page - 21

EFCLEAR

PROTOTYPE:

efclear(char *eflagh, int value)

ARGUMENTS:

eflagh         Pointer to event flag

value          Bits to clear in the event flag

RETURN VALUE:

none

DESCRIPTION:

This function clears bits in the event flag.  The existing value

of the event flag is anded with the complement of the value

parameter.  This cannot cause any tasks to become active.

EXAMPLE:

char *eflagh; /* Handle for eflag */

efclear (eflagh, 0xFFFF); /* clear all bit of event flag */

Page - 22

EFCLOSE

PROTOTYPE:

char *efclose(char *eflagh)

ARGUMENTS:

eflagh         pointer to the event flag

RETURN VALUE:

none

DESCRIPTION:

This function returns an event flag to the kernal.

EXAMPLE:

char *eflagh; /* Handle for eflag */

efclose(eflagh);

Page - 23

EFOPEN

PROTOTYPE:

char *efopen(int initial_value)

ARGUMENTS:

initial_value  Starting value of the event flag

RETURN VALUE:

handle to event flag data structure

DESCRIPTION:

This function obtains a handle to an event flag data structure

for use with the efwait() and efset() and efclear() functions.

The initial value can be in the range zero to 0xFFFF and each bit

is typically used to indicate a separate event.  A task can wait

on a particular event by providing a 16 bit mask that is anded

with the event flag.

EXAMPLE:

char *eflagh; /* Handle for eflag 1 */

if !(eflagh = efopen(0)) printf("Error opening Event Flag\n");

Page - 24

EFSET

PROTOTYPE:

efset(char *eflagh, int value)

ARGUMENTS:

eflagh         Pointer to event flag

value          Bits to set in the event flag

RETURN VALUE:

none

DESCRIPTION:

This function sets bits in the event flag.  The existing value of

the event flag is OR'ed with the value parameter.  Any tasks

waiting on the event flag are then evaluated and if any tasks are

able to run they are scheduled for execution.  Multiple tasks can

be activated simultaneously.  The actual task that executed will

depend on the relative priority.

EXAMPLE:

char *eflagh; /* Handle for eflag */

task1()

{

...

efwait(eflagh, 0x0001, 0);/*Wait for bit 1 ,no timeout */

...

}

efset (eflagh, 0xFFFF); /* set all bits of the event flag */

Page - 25

EFWAIT

PROTOTYPE:

efset(char *eflagh, int mask, int timeout)

ARGUMENTS:

eflagh         Pointer to event flag

mask           mask value to select which bits to wait for in

event flag

Timeout        Timeout value, if zero waits forever

RETURN VALUE:

value of event flag if task didn't timeout

0 if timeout

DESCRIPTION:

This function waits for the event flag to be set.  The mask is

anded with the current value of the event flag, if the result is

zero the task is put onto a list waiting for the event flag to be

set.  If either the event flag is set using efset() or the

timeout value expires, the task is removed from the waiting list

and resumed.  The return value is either the event flag value or

zero if a timeout occurred.

EXAMPLE:

char *eflagh; /* Handle for eflag */

task1()

{

...

efwait(eflagh, 0x0001, 0);/*Wait for bit 1 ,no timeout */

...

}

efset (eflagh, 0xFFFF); /* set all bits of the event flag */

Page - 26

FLUSHMSG

PROTOTYPE:

flushmsg()

ARGUMENTS:

none

RETURN VALUE:

none

DESCRIPTION:

This function discards all messages pending on the current task's

message queue.

EXAMPLE:

flushmsg();  /* Discard messages */

Page - 27

GETMSG

PROTOTYPE:

int getmsg(int timeout)

ARGUMENTS:

timeout        maximum time to wait for message (0=wait forever)

RETURN VALUE:

Value of message (Task blocked until message available).

0    Timed out

DESCRIPTION:

Retrieve next message of current task's message queue, if none

available the task is blocked until one is available or a timeout

occurs.  If a timeout value of zero is defined it will wait

forever.

The message can be sent by sendmsg() or defpoll().

EXAMPLE:

int message;

message = getmsg(20);/* Obtain next message, wait one second*/

Page - 28

GETTIME

PROTOTYPE:

int gettime()

ARGUMENTS:

None

RETURN VALUE:

Value of 68HC11 hardware timer.

DESCRIPTION:

Get the value of the hardware timer.  With an 8MHz clock this

timer does two counts per microsecond.  This function is useful

for timing very short events.  Since the maximum time is less

than one system tick, the timer will wrap between clock ticks.

EXAMPLE:

int i,j;

i=gettime();

...            /* Do function you wish to time */

j=gettime();

printf("Function takes %d microseconds\n",(j-i)/2);

Page - 29

GETTOD

PROTOTYPE:

int getTOD(char *time)

ARGUMENTS:

Pointer to 5 byte array to receive time of day.

RETURN VALUE:

Time of day.

DESCRIPTION:

MX-11 maintains a time of day clock with a resolution of a

twentieth of a second.  It operates as a twenty four hour clock

and rolls over at 23:59:59.  This function copies the time into

the array given as an argument.  The fifth byte contains the day

of the week with 0 meaning Sunday.

The time is incremented every twentieth of a second, and is

updated before tasks on the pollqueue are called.

time[0] = 1/20 seconds

time[1] = seconds

time[2] = minutes

time[3] = hours

time[4] = day of week ( 0=Sunday )

EXAMPLE:

char time[4];

getTOD(time);

printf("The time is %2d:%02d:%02d\n",time[3], time[2], time[1]);

Page - 30

MXINIT

PROTOTYPE:

char *mxinit(int stack_size, int numtcb, int numecb);

ARGUMENTS:

stack_size     number of bytes on stack to reserve for main().

numtcb         number of Task Control Blocks to create.

numecb         number of Event Control Blocks to create.

RETURN VALUE:

Pointer to Task Control Block for main().

DESCRIPTION:

The "mxinit" function initializes MX-11.  It must be executed

from main() before any other MX-11 functions are called.  It

allocates memory from the heap for the TCBs and ECBs, sets up the

task main() and nulltask().  Timer OC1 is used to provide a 20 Hz

real time interrupt.

Be careful about the amount of stack space that is set aside,

some library functions require large amounts for local variables.

e.g.. printf uses a 100 byte buffer.

MXSTKCHK can assist in avoiding stack overflow problems.

EXAMPLE:

char mainh; /* Handle for main */

mainh = mxinit(256, 8, 32); /* Provide for 8 tasks and 32 ECBs */

Page - 31

MXSTKCHK

PROTOTYPE:

mxstkchk()

ARGUMENTS:

none

RETURN VALUE:

none

DESCRIPTION:

This enables stack checking whenever a taskswitch occurs.  This

adds about 250 microseconds to each task switch.  It patches the

mxswch function so will only function correctly when loaded into

RAM.  If ROM use is desired the file mxkernal.asm should be

modified.

It should be invoked after mxinit().

When a task is defined the maximum stack extent is recorded in

the TCB, also a stack signature is placed at the maximum stack-1.

At every task switch all tasks are checked to verify whether

there is sufficient space to support the space needed for an

interrupt, and whether the stack signature has been corrupted.

If it has, a message is output via the SCI and execution

transfers to the debugger routine "monitor" as defined in MX.EQU

file.

EXAMPLE:

char mainh; /* Handle for main */

mainh = mxinit(256, 8, 32);/* Provide for 8 tasks and 32 ECBs */

mxstkchk();

Page - 32

SCLOSE

PROTOTYPE:

int sclose(char * semaphore_handle)

ARGUMENTS:

semaphore_handle    Pointer to semaphore

RETURN VALUE:

none

DESCRIPTION:

Returns a semaphore to kernel after completion of use.  Recycles

the data structure for other tasks.

EXAMPLE:

char *sema; /* Handle for semaphore */

...

if !(sema = sopen(0)) printf("Error opening Semaphore\n");

...

swait(sema); /* wait for it to be signaled */

...

sclose(sema); /* Not needed any more, recycle it */

Page - 33

SENDMSG

PROTOTYPE:

sendmsg (char *taskh, int msg)

ARGUMENTS:

taskh          task handle

msg            integer message to send to task

RETURN VALUE:

0 if error.

1 if no error.

DESCRIPTION:

This function puts a message on a task's message queue.  If the

task is dormant, the task is scheduled to be run with the message

as its input parameter.  Alternatively the task can obtain the

message by performing the getmsg() call.

Although the message is only a 16 bit variable, it can be used as

a pointer so data of any size can be passed.

EXAMPLE:

char *task1h; /* Handle for task1 */

task1(msg)

{

switch (msg)

{

...

}

}

task1h = deftask(&task1, 128, "Task One");

sendmsg (task1h, 127); /* Send value of 127 to task 1 */

Page - 34

SETTOD

PROTOTYPE:

int setTOD(char *time)

ARGUMENTS:

Pointer to 5 byte array containing time of day.

RETURN VALUE:

None.

DESCRIPTION:

MX-11 maintains a time of day clock with a resolution of a

twentieth of a second.  It operates as a twenty four hour clock

and rolls over at 23:59:59.  This function copies the time from

the array given as an argument.  The fifth byte gives day of the

week.

EXAMPLE:

char time[5];

/* Set time to Monday : 13:20 */

time[0]=0;

time[1]=0;

time[2]=20;

time[3]=13;

time[4]=1; /* Monday */

setTOD(time);

Page - 35

SLEEP

PROTOTYPE:

sleep( int timeout)

ARGUMENTS:

timeout        Value for timeout in clock ticks (1/20 second)

RETURN VALUE:

none

DESCRIPTION:

This function causes a pause in the execution of the calling

task.  Other tasks can still execute.

EXAMPLE:

sleep (20);    /* Cause a one second pause */

Page - 36

SOPEN

PROTOTYPE:

char *sopen(int initial_value)

ARGUMENTS:

initial_value  Starting value of the semaphore

RETURN VALUE:

handle to semaphore data structure

DESCRIPTION:

This function obtains a handle to a semaphore data structure for

use with the swait() and ssignal() functions.  The initial value

can be in the range zero to 32767 and determines how many

resources can be consumed before the task will be blocked.  If it

is set to zero the task will be blocked initially.

EXAMPLE:

char *sema1; /* Handle for semaphore 1 */

if !(sema1 = sopen(0)) printf("Error opening Semaphore\n");

Page - 37

SSIGNAL

PROTOTYPE:

void ssignal(char *semaphore_handle)

ARGUMENTS:

semaphore_handle    Pointer to semaphore

RETURN VALUE:

none

DESCRIPTION:

Increments the value of a  semaphore if no tasks are waiting on

the semaphore.  If any tasks are waiting in the semaphore, the

first one on the list is unblocked and the semaphore is left at

zero.

A semaphore can be used to provide mutual exclusion of use of a

resource.

EXAMPLE:

task1()

{

swait(sema1);   /* wait for serial I/O to be available */

printf("We don't want this to be interrupted by another task\n");

ssignal(sema1); /* Somebody else can use the serial I/O now */

}

Page - 38

SWAIT

PROTOTYPE:

int swait(char *semaphore_handle, timeout)

ARGUMENTS:

semaphore_handle    Pointer to semaphore

timeout             Timeout value in ticks, 0 means wait forever

RETURN VALUE:

1    Semaphore signaled

0    Timeout

DESCRIPTION:

MX-11 supports counting semaphores for inter-task

synchronization.  The "swait" function tests a semaphore

previously opened with "sopen", if it has a value greater than

zero, it is decremented and the task continues.  If the value is

zero, then the task is placed on the end of the list of tasks

waiting on the semaphore and the task is blocked.  Multiple tasks

can be waiting on a semaphore and will be unblocked in list order

as "ssignal" called.  If a timeout occurs swait() will return

zero.

A semaphore can be used to provide mutual exclusion of use of a

resource.

EXAMPLE:

task1()

{

swait(sema1,0);   /* wait forever serial I/O to be available */

printf("We don't want this to be interrupted by another task\n");

ssignal(sema1); /* Somebody else can use the serial I/O now */

}