PCM-4823 Utility Disk V1.1              2/27/1998

-----------------------------------------------------------------------------

The PCM-4823 is supplied with a software utility disk. This disk contains the

necessary file for setting up the VGA display and LAN controller.

Directories and files on the disk are as follows:

ROOT

�

���� README.DOC

���� 4823-SYS.BIN

���� 4823-STN.BIN

���� 4823-EL.BIN

���� 4823-MON.BIN

���� 4823-CRT.BIN

���� 4823-TFT.BIN

���� AWDFLASH.EXE

���� CBROM.EXE

AWDFLASH.EXE

------------

This program allows you to write the VGA display driver files to the BIOS

Flash ROM. The VGA files all come ready formatted for the PCM-4823 with .BIN

extensions. These files support various CRT and flat-panel displays. They

are custom written and can be made available upon request.

4823-SYS.BIN

------------

This binary file contains the system BIOS.

4823-TFT.BIN

------------

Supports 640x480 color 18/24-BIT TFT (Sharp LQ9D011, Toshiba LTM10C042)

4823-STN.BIN

------------

Supports 640x480 color STN DD 8/16-bit displays (Sharp LM64C142)

4823-MON.BIN

------------

Supports 640x480 dual scan monochrome displays (Sharp LM64183)

4823-EL.BIN

------------

Supports 640x480 EL displays (PLANAR EL640.480-A Series)

4823-CRT.BIN

------------

This binary file supports CRT display only.

CBROM.EXE

------------

This program allows you to combine your own VGA BIOS with system BIOS.

If you have your own VGA BIOS ( which supports special flat panel ) for

PCM-4823, please update your BIOS refer to the following example:

COPY 4823-SYS.BIN 4823-NEW.BIN

CBROM 4823-NEW.BIN /VGA NEW-VGA.DAT

AWDFLASH 4823-NEW.BIN

NEW-VGA.DAT : Your own VGA BIOS file (which supports special LCD) for PCM-4823.

4823-NEW.BIN: The NEW BIOS supports your own special LCD for PCM-4823.

-------------------------------------------------------------------------------