| Press Release ˙Press Release ˙Press Release ˙Press Release˙Press Release |  |
| --- | --- |
| Address: 1F, No. 6, Alley 6, Lane 45, Pao Hsin Road, Hsin Tien City, Taipei, Taiwan, R.O.C. TEL:886-2-2912-1234 FAX:886-2-2913-0792 E-mail: sales@mail.aaeon.com.tw http: //www.aaeon.com.tw Contact: Ms. Liwen Lin ext. 611 | PCM-7890 World’s first Pentium II® Little Board with on-board LCD controller, Fast Ethernet, Flash Disk, Sound, TV output and 4 COM ports. Date: July 25, 1998 |
|  | Introduction: With the new PCM-7890, we are continuously adding to our history of innovation. In an industry that increasingly requiring greater power, smaller computers, more onboard features, the PCM-7890 is truly a revolutionary product. The PCM-7890 is Little board with onboard CRT/LCD controller and the new DiskOnChip Flash Disk. The onboard 100BaseTx Fast Ethernet interface, TV output and Sound Blaster-compatible audio opens up worlds of possible applications for kiosks, gaming systems, POS systems, medical and educational products. This ultra compact board packs all the functions of an embedded PC on a single board, but fits in the space of a 5 1/4" floppy drive (only 5.75" x 8"). The on-board C&T 69000 is a high performance LCD/CRT Windows accelerator incorporated with 2MB SDRAM for graphics/video frame buffer. It supports a wide range of flat panel displays, including 36-bit TFT LCD displays allowing simultaneous viewing of LCD/CRT and TV display. The built-in DiskOnChip® supports system boot-up and memory storage up to 72 MB. The Fast Ethernet provides a tenfold increase in network capability; it is fully compatible with traditional 10Mbps network facilities. Hi-quality sound requirements are also provided for with an onboard PCI-bus ES1371 sound controller which is compatible with Sound Blaster, Sound Blaster Pro and Windows Sound System. The PCM-7890 also includes four high speed serial ports (three RS-232s, one RS-232/422/485), one multi-mode (ECP/EPP/SPP) parallel port, a floppy drive controller, an Ultra DMA/33 enhanced IDE controller and a keyboard/PS/2 mouse interface. The AAEON single board computer includes everything you need for fast, worry-free embedded PC performance. |