SG-104

Global Positioning System Receiver

Features

- PC 104 standard expansion module			Rockwell 12 Channels GPS engine

- Fast Time-To-First-Fix

- Can use either passive or active antenna for signal reception

- Differential with RTCM SC-104 input is ready for DGPS use

- RS-232 interface for PC communication

- RTC backup

- 1 PPS & 10K clock output with ±1ms error

General Description

The SG-104 GPS receiver module is designed with sophisticated satellite technology to locate and track any location in real time. It provides a wide range of GPS functionality such as position, navigation, satellites information, and timing information for the use of the embedded-PC navigation systems.

The SG-104 uses Rockwell’s 12 channels GPS engine which can track 12 satellites with “ ALL In View” , fast Time-To-First-Fix in maximum accuracy. Further more, The SG-104 is differential input ready for the use of the DGPS to assure pin-point accuracy.

Application

- automobile and/or marine navigation

- Automated vehicle tracking

- Fleet management

- Air navigation

- Emergency locator

- Time synchronization

Specification

Receiver  	15 m RMS(L1,C/A code) HDOP 	2.5 without SA.

Antenna	L1 frequency(1575.42MHz)

Output	Latitude, Longitude, Altitude

Time, Satellite Information, 	Speed, Time Mark (1PPS), 10K 	clock.

Input	RTCM-104 (9600,N,8,1)

Data Protocol 	NMEA  0183

Special Protocol(Custom design)

Acquisition 	20 Sec. (Warm )

2 Min. (Cold)

7 Min. (Auto Config.)

Accuracy	15 m without S/A

2-5m with DGPS RTCM 104

Power	5 Volts  280 mA

Data Update	1 Second (adjustable 1 to 5 	second)

Operation

Dimension	96mm X 91mm X 20mm

Operation Temp	-10°C to 60°C

Storage Temp	-30°C to 100°C

Humidity	95%@+60°C

Acceleration	4G

Velocity	950m/sec,3420Km/Hour

Altitude	-310 to 12,400 Meters

Jumper Setup

J1:

J1-1	NC

J1-2	DGPS, RTCM-104  ±12Volts level

J1-3	NC

J1-4 	10KHz clock when J24 is short

J1-5	Ground

J1-6	NC

J1-7	1PPS when J25 is short

J1-8	NC

J1-9	NC

J1-10	Vcc, +5Volts output

J10: short

1PPS :

___éù___  High = 25.7 ms

1PPS can use interrupt service program

J11 : IRQ 3

J12 : IRQ 4

J13 : IRQ 5

J14 : IRQ 7

J15 : IRQ 10

J16 : IRQ 11

J17 : IRQ 12

J18 : IRQ 14

J19 : IRQ 15

UART Com port & Interrupt

J20 : COM 1

J21 : COM 2

J22 : COM 3 (default)

J23 : COM 4

J4 : IRQ 3

J5 : IRQ 4

J6 : IRQ 5 (default)

J7 : IRQ 7

GPS

BUS

J1

BNC

* When led is not flash . Please send the board to  	manufacture . It may be have some problem.