3 ZODIAC NMEA DATA MESSAGES

This section describes the National Marine Electronics Association (NMEA) data messages of the Zodiac GPS receiver. All of the output and input NMEA messages are listed in Table III-1 together with their corresponding message IDs. Power-up default messages are also identified.

NMEA mode is selected according to the logic described in the hardware interface section of the Zodiac GPS Receiver Family Designer’s Guide. NMEA messages are transmitted and received across the host port serial I/O interface (RS-232) with the following default communications parameters:

4800 bps          ‧no parity

8 data bits         ‧1 stop bit

This interface conforms with the NMEA-0183, version 2.01, specification. All of the output NMEA messages are described in detail in section 3.1. All of the input NMEA messages are described in detail in section 3.2.

Table III-1, Zodiac NMEA Data Messages

Output Message Name

Message ID

GPS Fix Data (*)

GGA

GPS DOP and Active Satellites (*)

GSA

GPS Satellites in View (*)

GSB

Recommended Minimum Specific FPS Data (*)

RMC

Rockwell Proprietary Receiver ID

RID

Rockwell proprietary Zodiac Channel Status (*)

ZCH

- Input Message Name

- Message ID

- Rockwell Proprietary Log Control Message

- ILOG

- Rockwell Proprietary Receiver Initialization

- INIT

- Rockwell Proprietary Protocol Message

IPRO

(*) Default power-up message

3.1 Output Message Descriptions

Table III-2 . GGA Message: GPS Fix Data Message(1 of 2)

Message ID:

GGA (while receiver is in navigation Mode – Note 1)

Rate:

Variable; defaults to 1 Hz

Fields:

14

Field No:

Symbol:

Field Description:

Field Type:

Example:

$__GGA

Start of sentence and address field

$GPGGA

1

POS_ITC

UTC of position (hours, minutes, seconds, decimal seconds)

hhmmss.ss

234208.24

2

LAT

Latitude

| llll.ll 3339.685 3 LAT_REF Latitude direction (N= north, S= south) a N 4 LON Longitude yyyyy.yy |  | 11751.666 |
| --- | --- | --- |
| 5 LON_REF Longitude direction (E= east, W= west) a W 6 GPS_ QUAL GPS quality indicator (Note 2) x 1 7 NUM_SATS Number of satellites in use, 00 to 12 (may be different from the number in view) (Note 3) xx 04 8 HDOP Horizontal Dilution of Precision (HDOP) x.x 1.96 9 ALT_MSL Antenna altitude above/below mean sea level (geoide) x.x 28.9 10 M Units of antenna altitude (meters) M M 11 GROID_SEP Geoidal separatio |  |  |

n

| (Note 4) x.x -34.4 12 M Units of geoidal separation (meters) |  |  |
| --- | --- | --- |
| M M 13 DGPS_AGE Age of differential GPS data (Note 5) x.x (null) 14 STA_ID Differential reference station ID (0000 to 1023)(Note 6) xxxx (null) CKSUM Checksum *hh *54 <CR><LF> Sentence terminator <CR><LF> Example Message: $GPGGA, 234208.24,339.685,N,11751.6 |  |  |

66,W,1,04,1.96,28.9,M,-34.4,M,,*54

Table III-2. GGA Message : GPS Fix Data Message (2 of 2)

Note 1:

When the receiver is in Acquisition Mode, fields through 5 and 8 through 14 ar

| NUM-SATSTotal number of messages (1 to 3)e null. Field 7 also has special meaning (see Note 3). Total number of satellites in view x 2 2 NUM_MSG Message number (1 to 3) x 1 3 Note 2: GPS quality indicator: 0=Fix not available or invalid. 1=GPS fix. 2=Differential GPS fix. Note 3: When the receiver is in Acquisition Mode, this field represents the number of satellites currently being tracked. Note 4: Geoidal separation is the difference between the WGS_84 Earth ellipsoid and means sea level (geoid). Note 5: Time in seconds since the last SC104 Type 1 or Type 9 update; null field when DGOS is not used. Note 6: This field is null when DGPS is not used. Table III-3. GSA message: GPS DOP and Active Satellites Message Message ID: GSA Rate: Variable Fields: 17 Field No.: Symbol: Field Description: | Field Type: Example: $__GSA Start of sentence and address field $GPGSA 1 OP_MODE Mode (Note 1) a A 2 FIX_MODE Mode (Note 2) x 3 3-14 SATN PRNs of satellites used in solution (null for unused fields) xx,xx,… 04,16,09,24,… 15 PDOP Position dilution of Precision (PDOP) (Note 3) x.x 3.33 | 16 HDOP Horizontal Dilution of Precision (HDOP) (Note 3) x.x 1.96 17 VDOP Vertical Dilution of Precision (VDOP) (Note 3) x.x 2.70 |  | CKSUM Checksum | *hh *06 <CR><LF> Sentence terminator | <CR><LF> Note 1: Mode (operating): M= Manual, forced to operate in 3-D mode. A= Automatic, allowed to automatically switch between 2-D and 3-D. Note 2: Mode (fix): 1= Fix not available 2= 2-D 3= 3-D Note 3: DOPs are based on the set of satellites above the elevation mask angle, which may not be the same set as that used for navigation. Example Message: $GPGSA,A,3,04,16,09,24, , , , , , , , , 3.33, 1.96,2.70*06 Table III-4. GSV Message: GPS Satellites in View Message Message ID: GSA Rate: Variable; defaults to 0.5 Hz Fields: 19 Field No.: Symbol: Field Description: Field Type: Example: $__GSA Start of sentence and address field |
| --- | --- | --- | --- | --- | --- | --- |
| 234215.24RMC(while receiver is in Navigation Mode—Note 1) xx$GPGSV 3 LAT Latitude IIII.II 3339.686 4 LAT_REF Latitude direction (N= north, S= south) a N 5 LON Longitude yyyy.yy 11751.667 6 LON_REF Longitude direction (E= east, W= west) a W 7 SPD Speed over ground (knots) x.x 0.620 8 HDG Heading/track made good (degrees True) x.x 293.8 9 DATE Date(dd/mm/yy) xxxxxx 180594 10 MAG_VAR Magnetic variation(degrees) x.x 14.0 11 MAG_REF Magnetic variation (E= east, W= west) (Note 3) a E CKSUM Checksum *hh *79 <CR><LF> Sentence terminator <CR><LF> Note 1: When the receiver is in Acquisition Mode, field 1 and fields 3 through 11 are null. Note 2: The position status flag will be set to “V” (data invalid) until the receiver is navigating. At that time, the 2 POS_STAT Position status(A=Data valid, V=Data invalid (Note 2) a A Rate: Variable; defaults to 1 Hz Fields: 11 Field No.: Symbol: Field Description: Field Type: Example: $__RMC Start of sentence and address field $GPRMC 1 POS_UTC UTC of position (hours, minutes, seconds, decimal seconds) hhmmss.ss 07 4 SAT_PRN Satellite PRN number (Note 1)) xx 24 5 ELEV Elevation in degrees (90 degrees maximum) (Note 2) xx 60 1 MAX_MSG | 6 AZ Azimuth in True degrees (000 to 359) (Note 2) xxx 216 7 SNR | SNR(C/No) 00 to 99 dB, null when not tracking xx 50 8-11 … 2nd satellite PRN number, elevation, azimuth, SNR (Note 1) xx,xx,xxx,xx … 12-15 … 3rd satellite PRN number, elevation, azimuth, SNR (Note 1) |  | xx,xx,xxx,xx … 16-19 | … 4th satellite PRN number, elevation, azimuth, SNR (Note 1) xx,xx,xxx,xx … | CKSUM |
| Checksum *hh *75 <CR><LF> Sentence terminator <CR><LF> Note 1: The visible satellites may include one or more that are below the horizon. Since NMEA does not account for negative elevation angles, the elevation field will be null for these satellites. Note 2: Azimuth and elevation are null when the satellite is in track, but a visible list is not available. Example message: $GPGSV,2,1,07,24,60,216,50,20,47,135,47,12,40,020,47,16,36,319,46*75 Table III-5. RMC Message: Recommended Minimum Specific GPS Data Message Message ID: |  |  |  |  |  |  |

flag is changed to “A” (data valid) and the information provided in the RMC message will reflect a navigation solution.

Note 3:

Easterly variation (E) subtracts from True course.

Westerly variation (W) adds to True course.

Example Message:

$GPRMC,234215.24,A,3339.686,N,11751.667.W,0.620,293.8,180594,14.0,E*79

Table III-6. RID Message: Rockwell Proprietary Receiver ID Message

Message ID:

RID

Rate:

Variable (see above)

Fields:

5

Field No.:

Symbol:

Field Description:

Field Ty

| pe: Example: $____RID | Start of sentence and address field $PRWIRID 1 NUM_CHN Number of Channels xx 12 |  |
| --- | --- | --- |
| 5RES Reserved CKSUM Checksum *hh *40 <CR><LF> Sentence terminator <CR><LF> Note 1: The options list is a bit-encoded configuration word represented as a four-digit hexadecimal number. bit 0 minimize ROM usage bit 1 minimize RAM usage bits 2-15 reserved Sample Message: $PRWIRID,12,00.90,12/25/95,0003,*40 Table III-7. ZCH Message: Rockwell Proprietary Zodiac Channel Status Message Message ID: ZCH Rate: Variable; defaults to 1 Hz 2 SW_VER Software Version x.x 00.90 3 SW_DATE Software Date cccccccc 12/25/95 4 OPT_LST Options List (Note 1) hhhh 0003 | Fields: 24 Field No.: Symbol: Field Description: Field Type: Example: $__ZCH Start of sentence and address field |  |
| $PRWIZCH 1-2 SAT_PRN Channel 1 satellite PRN number (Note 1) xx 24 2 STATUS Channel 1 status indication (Note 1) hh__ 7 3-4 … channel 2 satellite PRN number and status indication xx,hh__ … 5-6 … channel 3 satellite PRN number and status indication xx,hh__ … 7-8 … channel 4 satellite PRN number and status indication xx,hh__ … 9-10 … channel 5 satellite PRN number and status indication xx,hh__ |  |  |

...

11-12

…

channel 6 satellite PRN number and status indication

xx,hh__

…

13-14

…

channel 7 satellite PRN number and status indicationxx,hh__

…

15-16

…

channel 8 satellite PRN number and status indication

xx,hh__

…

17-18

…

channel 9 satellite PRN number and status indication

xx,hh__

…

19-20

| … channel 10 satellite PRN number and status indication xx,hh__ … 21-22 … channel 11 satellite PRN number and status indication xx,hh__ … 23-24 … channel 12 satellite PRN number and status indication xx,hh__ … CKSUM Checksum xx,hh__ … <CR><LF> Sentence terminator *hh *64 Note 1: Channel number (xx) is implied by position in message. Data for all 12 channels is always provided in this message. If a channel is unused, a value of 0 will appear for both channel fields. The status indication (hh__) is a one-digit, hexadecimal value which represents four bits as follows: 0> Measurement of the satellite on this channel used in navigation solution. <y.1> Ephemeris available for the satellite on this channel. 2> Satellite on this channel is in track. <y.3> DGPS corrections available for the satellite on this channel (NOTE: this bit will never be set whenever the configuration of a particular Zodiac GPS receiver does not support DGPS.) Example Message: $PRWIZCH,20,0,21,2,22,4,23,6,24,7,25,7,26,7,27,7,28,7,29,7,30,7,31,6*64 3.2 Input Message Descriptions Table III-8. ILOG Message: Rockwell Proprietary Log Control Message Message ID: ILOG Rate: As required Fields: 5 Field No.: Symbol: Field Description: Field Type: Example: $PRWIILOG Start of sentence and address field (Note 1) $PRWIILOG 1 MSG_ID Approved sentence formatter of the data being requested (Note 2) ccc RMC 2 ENABLE Output enables flag(A=enable, V=disable) (Note 3) a A 3 TRIG Output trigger (t=on time, u=on update) (Note 4) a T 4 INTERVAL Output interval (seconds, 0=once) (Note 4) x 5 5 OFFEST Initial output offset (seconds from minute mark)(Note 4) x 0 CKSUM Checksum (optional) *hh | <CR><LF> Sentence terminator <CR><LF> Note 1: $ = NMEA message prefix. P = Proprietary message indicator. RWI = Rockwell International mnemonic. ILOG = Log control message ID. Note 2: A special form of this field disables all output messages. Use “???” as the message ID as in the following example: $PRWIILOG,???,V , , , Note 3: This field may be null to indicate that the previous setting should be left unchanged. Note 4: The TRIG, INTERVAL, and OFFSET fields may be null to indicate that the previous setting should be left unchanged. Example Message: $PRWIILOG, RMC, A,T, 5, 0 Table III-9. INIT Message: Rockwell Proprietary Receiver Initialization Message(1 of 2) Message ID: INIT Rate: As required Fields: 14 Field No.: Symbol: Field Description: Field Type: Example: $PRWIINIT Start of sentence and address field (Note 1) $PRWIINIT 1 RESET Software reset flag (A=reset, V=don’t reset)(Note 2) a V 2 RES_1 Reserved 3 RES_2 Reserved 4 LAT Latitude (Note 2) IIII.III 3339.650 5 LAT_REF Latitude direction (N=north, S= south) (Note 2) a N 6 LON Longitude (Note 2) yyyyy.yy 11751.680 7 LON_REF - Longitude direction (E=east, W= west) (Note 2) - a - W - 8 - ALT - Altitude (meters)(Note 2) - x.x - 64.131 - 9 - SPD - Ground speed (Note 2) - x.x - 0.0 - 10 - SPD_TYP - Ground speed units (M=m/sec, N=knots, K=kn/hr) (Note 2) a M 11 HDG Heading (0.0 to 360.0 degrees north) (Note 2) xxx.x 0.0 12 HDG_TYP Heading type (T= true, M= magnetic) (Note 2) a T 13 TIME UTC time (hours, minutes, seconds) (Note 2) hhmmss 162338 14 DATE UTC date (Note 2) ddmmyy 190594 CKSUM Checksum (optional) *hh <CR><LF> |  |
| --- | --- | --- |
| sentence terminator <CR><LF> Table III-9. INIT Message: Rockwell Proprietary Receiver Initialization Message (2 of 2) Note 1: $ =NMEA message prefix. P =Proprietary message indicator. RWI =Rockwell International Mnemonic |  |  |

.

INIT   =Initialization message ID.

Note 2:

This function is enabled by default.

Each of the fields 1 through 14 may be null to indicate that the previous setting for the data item should be left unchanged. For example, reset may be commanded without specifying the other parameters by issuing the following command:

$PRWIINIT,A, , , , , , , , , , , , , , , <CR><LF>

When using null fields, the following restrictions apply:

．If a supplied parameter has a corresponding unit specifier or reference indicator, it must also be supplied.

．Both latitude and longitude must be provided to specify a valid horizontal position.

．Both ground speed and heading must be provided to specify a valid horizontal velocity.

．If a magnetic heading is specified, horizontal position (lat/lon), and UTC time and date also be provided.

．UTC time and date must be provided together.

Example Message:

$PRWIINIT, V, , , , 3339.650, N, 11751.680, W, 64.131, 0.0, M, 0.0, T, 162338, 190594

Table III-10. IPRO Message: Rockwell Proprietary Protocol Message

Message ID:

IPRO

Rate:

As required

Fields:

2

Field No.:

Symbol:

Field Description:

Field Type:

Example:

$PRWIIPRO

Start of sentence and address field (Note 1)

$PRWIIPRO

1

RES

Reserved (Data Stream Select) (HOST=host, AUX= auxiliary) (Note 1)

2

PRO_TYPE

Protocol Type (RBIN=Rockwell binary, RTCM=RTCM SG-104

cccc

RBIN

CKSUM

Checksum (optional)

*hh

<CR><LF>

sentence terminator

<CR><LF>

Note 1:

RTCM SC-104 is not a valid protocol for the host data stream.

Example Message:

$PRWIIPRO. . RBIN