655XX VGA

Flat Panel/CRT Controller

Display Drivers

and Utilities

User's Guide

Release 2.6

March 1994

Trademarks

AutoCAD and AutoShade are trademarks of Autodesk, Inc.

CHIPS, and CHIPSCPL are trademarks of Chips and Technologies, Inc.

IBMÒ, OS/2 and VGA are trademarks of International Business Machines Corporation.

Lotus, 1-2-3 and Symphony are trademarks of Lotus Development Corporation.

TurboDLD Classic is a trademark of Panacea Inc.

Windows, Word, MS-DOS and Microsoft are trademarks of Microsoft Corporation.

WordPerfect is a trademark of WordPerfect Corporation.

VESA is a trademark of Video Electronics Standards Association.

TABLE OF CONTENTS

µChapter 1	4

Software Drivers	4

Windows	9

OS/2	12

AutoCAD	13

Lotus 1-2-3	16

Lotus Symphony	16

VESA Super VGA Driver	18

Word	20

WordPerfect	21

Chapter 2	22

Software Utilities	22

The CHIPSCPL utility program	23

The SETCOL utility program	25

.End Table C.

Chapter 1tc "Chapter 1" \l 1§

SOFTWARE DRIVERStc "Software Drivers" \l 2§

This chapter describes the operation and installation of the software drivers supplied on the Display Driver Diskettes that are shipped with your VGA adapter.

Your VGA adapter is based on the CHIPS VGA Flat Panel/CRT controller and is fully IBM VGA compatible.  This controller offers a large set of extended functions and higher resolutions.  If you intend to use your VGA adapter in standard VGA modes only, you do not need to install any of these drivers.  Since your VGA adapter is fully compatible, it does not require any special drivers to operate in standard modes.

The purpose of the enclosed software drivers is to take advantage of the extended features of the CHIPS VGA Flat Panel/CRT controller.  These capabilities include:

n	High performance in Microsoft Windows

n	Resolutions up to 1280x1024 in graphics modes with 16 colors

n	Resolutions up to 1024x768 in graphics modes with 256 colors

n	Resolutions up to 640x480 in graphics modes with 32K, 64K and 16M colors

n	132 column text mode

The Windows drivers included in this package have several special features, which are described in the Windows section of this chapter:

n	Pen Windows Extensions

n	Panning Drivers

n	Linear Acceleration Drivers

Hardware Configuration

Some of the high-resolution drivers provided in this package will work only in certain system configurations.  If a driver does not display correctly, try the following:

1.	Change the display controller to CRT-only mode, rather than flat panel or simultaneous display mode.  Some high-resolution drivers will display correctly only in CRT mode.

2.	If a high-resolution mode is not supported on your system, try using a lower-resolution mode.  For example, 1024x768 mode will not work on some systems, but 800x600 mode is supported on most.

3.	The 65510 Flat Panel VGA Controller supports only those drivers that can display on a 640x480 panel.  See the following section on Driver Resolutions and Filenames for a list of the drivers that can be used on the 65510.

4.	The 65535 and 65540 Flat Panel VGA Controllers support additional drivers that function with the F65535 and F65540 only.  See the following section on Driver Resolutions and Filenames for a list of these drivers.

These software drivers support the following software applications in the filenames and resolutions listed:

Application	Filename	Resolution	Colors	Notes

Windows 3.1	LINEAR4.DRV	640x480	16	510
			800x600	16
			1024x768	16
			1280x1024	16
		LINEAR8.DRV	640x480	256	510
			800x600	256
			1024x768	256	535/540
		LINEAR15.DRV	640x480	32K	535/540
		LINEAR16.DRV	640x480	64K	535/540
		LINEAR24.DRV	640x480	16M	540
		W3LM600P.DRV	800x600	16
		W3LM768P.DRV	1024x768	16
		W3PM480.DRV	640x480	256	510
		W3PM600P.DRV	800x600	256
		R12.DRV	480x640	16	510

OS/2 Version 2.1	SV480256.DLL	640x480	256
		PD600256.DLL	800x600	256
		PD768256.DLL	1024x768	256

AutoCAD	DLDVGA.EXE	640x480	16	510
			800x600	16
			1024x768	16
			640x480	256	510

Lotus 1-2-3	V132X25.DRV	132x25(text)	16
		V132X50.DRV	132x50(text)	16

VESA SuperVGA	VESA.COM	800x600	16
			1024x768	16
			640x480	256

Word 5.0	VGA600.VID	800x600	16
		VGA768.VID	1024x768	16

Word 5.5	VGA55600.VID	800x600	16
		VGA55768.VID	1024x768	16

WordPerfect 5.0	CHIPS600.WPD	800x600	16
		CHIPS768.WPD	1024x768	16

WordPerfect 5.1	VGA600.VRS	800x600	16
		VGA768.VRS	1024x768	16

Notes:	510		Can be used on the 65510 controller.

535/540	Functions with the 65535 and 65540 controllers only.

540		Functions with the 65540 controller only.

Notational conventions

Certain conventions are used to describe the commands entered by the user and the response from the computer.  The following conventions are used throughout the manual:

Commands entered by the user are shown in a boldface font. Example:

COPY  A:*.*  C:

At the end of each line entered by the user, the Enter key should be pressed.  This is indicated by the following symbol:

<ENTER>

Sometimes the user is required to press certain function keys.  This is indicated by enclosing the function key name in brackets.  Examples:

<F10>		(the F10 key)

<END>		(the "End" key)

<HOME>	(the "Home" key)

Output from the system to the user is shown in a normal font.  Example:

C:\123

Menu alternatives are shown in a boldface italic font.  Example:

Option 1

Filenames and program names are also shown in a boldface italic font.  Example:

AUTOEXEC.BAT

Necessary prerequisites

The instructions in this manual assume that you understand elementary concepts of MS-DOS and the IBM Personal Computer.  Before you attempt to install any driver or utility, you should:

n	Know how to copy files from a floppy disk to a directory on the hard 	disk

n	Understand the MS-DOS directory structure

n	Know how to format a floppy disk

If you are uncertain about any of these concepts, please refer to the DOS or OS/2 user reference guides for more information, before you proceed with the installation.

Before you begin

Before you begin the driver software installation, you should make backup copies of the Display Driver Diskettes.  Store the original disks in a safe place.

Make sure you know the version of the application for which you are installing drivers.  Your Display Driver Diskettes contain drivers for several versions of certain applications.  For your driver to operate properly, you must install the driver for your version of the application program.

Windowstc "Windows" \l 3§

These drivers are designed to work with Microsoft Windows Version 3.1 and Pen Windows Version 3.1.

Driver installation

Step 1:	Install Windows as you normally would for a VGA display.  Run Windows to make sure that it is working correctly.  Then exit from Windows.

Step 2:	Place the Display Driver Diskette #1 in drive A.  Type A:<ENTER> to make this the default drive.  Type SETUP <ENTER> to run the driver SETUP program.  Press any key to get to the applications list.  Using the arrow keys, select Windows Version 3.1 and press the  <ENTER> key.  Press the <ENTER> key to select All Resolutions, then press <END> to begin the installation.  At this point you will be asked for the path to your Windows System directory (default C:\WINDOWS).  When the installation is complete, press any key to continue.  Press <ESC> followed by Y to exit to DOS.

Step 3:	Change to the directory where you installed Windows (usually C:\WINDOWS).

Step 4:	Type SETUP <ENTER> to run the Windows Setup program.  It will show the current Windows configuration.  Use the up arrow key to move to the Display line and press <ENTER>.  A list of display drivers will be shown.  Use the arrow keys to select one of the drivers starting with an asterisk (*) and press <ENTER>.

Step 5:	Follow the directions on the screen to complete the setup.  In most cases, you may press <ENTER> to accept the suggested option.  When Setup is done, it will return to DOS.  Type WIN <ENTER> to start Windows with the new display driver.

Changing Display Drivers from DOS

To change display drivers from DOS, change to the Windows directory and run Setup, repeating steps 4 and 5 from the previous page.  Besides the special display drivers marked by an asterisk (*), you should be able to use the following standard drivers:

VGA	640x480, 16 colors

Super VGA	800x600, 16 colors

Changing Display Drivers from Windows

To change display drivers from Windows, select the Windows Setup icon from the Main window.  You will be shown the current setup configuration.  Select Change System Settings from the Option menu.  Click on the arrow at the end of the Display line.  You will be shown a list of display drivers.  Click on the driver you want to select it.  Then click on the OK button.  Follow the directions to complete the setup.

Changing Color Schemes

After you change display drivers, you may notice that the color scheme used by Windows looks strange.  This is because different drivers have different default colors.  You can correct this by choosing the same color scheme or a new color scheme.  First select the Control Panel from the Main window.  Select the Color icon.  You will be shown the current color scheme.  Choose a new color scheme and click the OK button.

Panning Drivers

Special panning drivers are provided to allow high-resolution modes to be displayed on a flat panel or CRT.  These drivers will show a section of a larger screen, and will automatically pan or scroll the screen horizontally and vertically when the mouse reaches the edge of the display.

Linear Acceleration Drivers

A special high-performance linear acceleration driver is provided for 256-color modes.  This driver may require special hardware and may not be supported on all systems.  This driver is only available for Windows 3.1.  One feature that is quoted is QuickTime-Aware.

OS/2tc "OS/2" \l 3§

These drivers are designed to function with the OS/2 2.1 operating system.

Driver installation

NOTE:	Always use the INSTALL.CMD for  the FIRST installation of the video device drivers.  To change video resolutions, use the DSPINSTL program.

To install the drivers, follow these instructions:

Step 1:	Install OS/2 as you normally would for a VGA display.  Open an OS/2 full-screen or window session.

Step 2:	Place the Display Driver Diskette #3 in drive A.  Type A:<ENTER> to make this the default drive.  Then type INSTALL  A:  C:  <ENTER>.  Once the drivers are installed, the DSPINSTL program is automatically invoked.  Follow the instructions of the program to set up the OS/2 drivers in your system.  Once the installation is complete, the system must be shut down and restarted for changes to take effect.

Driver diskette copy

NOTE:	Diskette copies of the OS/2 drivers must have a VOLUME LABEL that reads "DISP 1" in order to be an installable diskette.

To copy the OS/2 Display Driver Diskette #3, follow these instructions:

Step 1:	Copy all files on the Display Driver Diskette #3 as you normally would onto another diskette.

Step 2:	Place the diskette copy in drive A.  At the C:\ prompt, type LABEL A: DISP 1 to properly label your diskette.  Then store your diskette copy in a safe place.

For proper installation of OS/2 drivers, all diskette copies must be properly labeled "DISP 1".

AutoCADtc "AutoCAD" \l 3§

These drivers are designed to work with Autodesk AutoCAD and AutoCAD/386 Release 10 and 11, and corresponding versions of AutoShade.  They conform to the Autodesk Device Interface (ADI) for Rendering drivers and Display drivers.  These display list drivers accelerate redraw, pan, and zoom functions.

Driver installation

To install the drivers, follow these instructions:

Step 1:	Place the Display Driver Diskette #1 in drive A.  Type A:<ENTER> to make this the default drive.  Type SETUP <ENTER> to run the SETUP program.  Press any key to get to the applications list.  Using the arrow keys, select AutoCAD & AutoCAD/386 Release 10 & 11 and press <ENTER>.  This will display a list of supported driver resolutions.  Press the <ENTER> key to select All Resolutions, then press <END> to begin the installation.  At this point you will be asked for a drive and directory to copy the driver files.  Enter the drive and directory that contains the installed AutoCAD.  You may be asked to change display driver diskettes, and if the destination directory does not exist you will be asked for confirmation.  When the installation is complete, press any key to continue.  Press <ESC> followed by Y to exit to DOS.

Step 2:	Go to the autocad directory where the new drivers were installed and run the driver installation program by typing DLDSETUP <ENTER>.  This program will configure your new fast display driver and create a batch file called FASTACAD.BAT.

Step 3:	Configure the colors for AutoCAD by using one of the following:	COLOR16 <ENTER>	if 16-color mode is used,

COLOR256 <ENTER>	if 256-color mode is used.

Start by pressing A to select the standard AutoCAD ADI colors.  Press S to save these colors in DLDCOLOR.DAT.  For more information about these color utilities, see Changing Colors later in this section.

Step 4:	Prior to starting an AutoCAD or AutoShade session, the display driver must be installed in the computer memory.  To do so, execute the batch file created by typing FASTACAD <ENTER>.  To remove the display driver from memory when you are finished using AutoCAD, you may use DLDVGA -U <ENTER>.

Configuring AutoCAD

The first time you use AutoCAD with the ADI driver, you need to configure AutoCAD for an ADI display.  Select CONFIGURE AUTOCAD from the AutoCAD main menu.  After you see the current configuration (if any) select CONFIGURE VIDEO DISPLAY.  Select ADI as your new driver (installed at interrupt 7A hex).  Save the new configuration and return to the main menu.

Configuring AutoShade

The first time you use AutoShade with an ADI driver, you need to configure AutoShade for an ADI display.  Before you run the AutoShade program, remove the file SHADE.CFG (use the DOS DELETE command if needed).  Then run the AutoShade program.  The program will ask for a display and rendering driver.  Select ADI for both drivers and indicate that you have a dual display system.  When you exit the AutoShade program, a new SHADE.CFG will be created.

Changing Colors

To change the colors that AutoCAD uses for drawing, background, menus and text, use the utility COLOR16 (16-color modes) or COLOR256 (256-color modes).  When running these utilities, use the ? key at any time to get context-sensitive help messages explaining how to use the program.

For example, to change the background color used by AutoCAD in 16-color mode, run the utility COLOR16.  Press O for object.  Use the left and right arrows to move through the different objects (shown at the top of the screen) until Graphics Area Background appears.  Press <ENTER> to select this object.  The current color will be shown at the bottom of the screen.  Use the left and right arrows to move to a new color and press <ENTER> to select this color.

When you are done changing objects, press <ESC> to exit to the main menu.  Press S to save these colors in the file DLDCOLOR.DAT.  Then press E to exit.

Lotus 1-2-3tc "Lotus 1-2-3" \l 3§

Lotus Symphonytc "Lotus Symphony" \l 3§

These drivers are designed to work with Lotus 1-2-3 Version 2.0, 2.01 and 2.2, and with Lotus Symphony Version 1.0 and 1.1.

Driver installation

To install the Lotus drivers, follow this procedure:

Step 1:	Place Display Driver Diskette #1 into drive A.  Make A the default drive by typing A:<ENTER>.  Run the SETUP program by typing SETUP <ENTER>.  Press any key to display a list of supported applications.  Use the arrow keys to select Lotus/Symphony and press <ENTER>.  A list of supported screen resolutions will be displayed.  Use the arrow keys to select the desired screen resolution and press <ENTER> (make sure your monitor is able to display the resolution desired).  Press <END> to begin the driver installation process.  A default drive and directory path will be displayed.  Use the backspace key to erase this default and type in the 123 directory.  At this point you may be prompted to insert one of the other driver diskettes.  You also may be asked to create the target directory if it does not already exist.  After the files have been installed, press any key to return to the list of supported applications.  Press <ESC> followed by Y to exit to DOS.  Copy all the files that were just created on the temporary directory onto a formatted floppy diskette.

Step 2:	Go to your 123 directory and start the installation program.  Type the following commands:

C: <ENTER>

INSTALL <ENTER>

Step 3:	The Lotus installation program will load and present the installation menu.  From this menu, select ADVANCED OPTIONS.  From the Advanced options menu, select ADD NEW DRIVERS TO LIBRARY.  From the Add New Drivers Menu, select MODIFY CURRENT DRIVER SET.  From the Modify Driver Set Menu, select TEXT DISPLAY.  From the text display menu, select one of drivers.

Step 4:	After the selection of the appropriate VGA display driver, you will need to exit this menu and return to the Main Lotus Installation Menu.  Do this by selecting RETURN TO MENU.

Step 5:	At the Main Lotus Installation Menu, select SAVE CHANGES.

Step 6:	At this point the Installation Menu will prompt you for the name of your new Lotus configuration file.  The Lotus system will prompt you with the default value -- 123.SET.  You do not have to use this name, however.  You may want to use a filename that indicates the resolution of the driver it contains.  For example, if you installed the 132 column by 25 line driver, you could name this driver 132X25.SET.  Or if you installed the 80 by 50 driver, you may want to call the file 80X50.SET.

Step 7:	The installation of your Lotus 1-2-3 driver is now complete.  You will need to exit the Lotus installation program at this point.  At the main Lotus Installation Menu, select EXIT.

Note:	If you use a different name of the driver set than 123.SET, you have to remember to place the filename of your driver set on the command line when you start Lotus 1-2-3.  For example, if you named your driver set 132X25.SET, give the following command to start Lotus 1-2-3:

123  132X25.SET <ENTER>

VESA Super VGA Drivertc "VESA Super VGA Driver" \l 3§

The Video Electronics Standards Association (VESA) has created a standard for a Super VGA BIOS Extension (VBE).  This defines a standard software interface to allow application program to set and control extended video modes, such as 800x600 graphics, on video adapters from different manufacturers.

The VESA driver adds this Super VGA BIOS Extension to the VGA BIOS.  Any application program which supports the VESA standard driver interface can be used with this driver.  This VESA driver conforms to the VESA Super VGA Standard #VS891001.

Driver installation

Step 1:	Place the Display Driver Diskette #1 into drive A.  Make A the default drive by typing A:<ENTER>.  Run the SETUP program by typing SETUP <ENTER>.  Press any key to display a list of supported applications.  Use the arrow keys to select VESA Driver Version 1.2 and press <ENTER>.  Press the <ENTER> key to select All Resolutions, then press <END> to begin the installation.  A default drive and directory path will be displayed.  Use the backspace key to erase this and type in a directory that is in the directory path (such as C:\BIN or C:\UTILS).  At this point you may be prompted to insert one of the other driver diskettes.  After the files have been installed, press any key to return to the list of supported applications.  Press <ESC> followed by Y to exit to DOS.

Step 2:	To install the VESA driver, type either VESA <ENTER> or VESA + <ENTER> at the dos prompt.  The optional + command line parameter enables all of the available modes.  Make sure that your monitor is capable of displaying these high resolution modes before enabling them.

NOTE:	If the video BIOS already supports VBE extended video modes, DO NOT install this driver.  Run the VTEST.EXE program (provided in Disk 2) to see if the video BIOS supports the VBE modes.

Wordtc "Word" \l 3§

These drivers are designed to work with Microsoft Word Version 5.0 and 5.5.

Driver installation

If you have already installed Word on your computer go to Step 2 to install the new video driver.

Step 1:	Install Word as you normally would.

Step 2:	After you complete the Word installation, place the Display Driver Diskette #1 into drive A.  Make A the default drive by typing A:<ENTER>.  Run the SETUP program by typing SETUP <ENTER>.  Press any key to display a list of supported applications.  Use the arrow keys to select Word and press <ENTER>.  Use the arrow keys to select the desired screen resolution and press <ENTER> (make sure your monitor is able to display the resolution desired).  Press <END> to begin the driver installation process.  A default drive and directory path will be displayed.  Use the backspace key to erase this and type in your Word directory.  At this point you may be prompted to insert one of the other driver diskettes.  After the files have been installed, press any key to return to the list of supported applications.  Press <ESC> followed by Y to exit to DOS.

Step 3:	Copy the driver file for the desired resolution that was just installed to SCREEN.VID.  (The driver file names are listed on page 9.)

WordPerfecttc "WordPerfect" \l 3§

These drivers are designed to work with WordPerfect Version 5.0 or 5.1.  They support 132-column display in editing mode, and high-resolution graphics display in PreView mode.

Driver installation

Step 1:	Place Display Driver Diskette #1 into drive A.  Make A the default drive by typing A:<ENTER>.  Run the SETUP program by typing SETUP <ENTER>.  Press any key to display a list of supported applications.  Use the arrow keys to select WordPerfect and press <ENTER>.  A list of supported screen resolutions will be displayed.  Use the arrow keys to select the desired screen resolution and press <ENTER> (make sure your monitor is able to display the resolution desired).  Press <END> to begin the driver installation process.  A default drive and directory path will be displayed.  Use the backspace key to erase this default and type in the WordPerfect directory.  At this point you may be prompted to insert one of the other driver diskettes.  You also may be asked to create the target directory if it does not already exist.  After the files have been installed, press any key to return to the list of supported applications.  Press <ESC> followed by Y to exit to DOS.

Step 2:	Start WordPerfect and press <SHIFT>+<F1> to enter the setup menu.  Select D for DISPLAY and G for Graphics SCREEN TYPE and then choose the desired Chips VGA resolution.

Configuring WordPerfect 5.0 for 132 columns

Follow these instructions to configure WordPerfect 5.0 for 132 column text mode:

Step 1:	Use the SETCOL program to set 132 columns and 25 rows.  Give the following command:

SETCOL  132, 25 <ENTER>

Step 2:	Start WordPerfect.  The program will detect the number of rows and columns automatically.  If for some reason WordPerfect is unable to adapt to 132 columns by 25 rows, start WordPerfect with the following command:

WP  /SS=25,132 <ENTER>

Configuring WordPerfect 5.1 for 132 columns

Start WordPerfect and press <SHIFT>+<F1> to enter the setup menu.  Select D for DISPLAY and T for TEXT SCREEN TYPE and then select Chips 132 Columns Text.

Chapter 2tc "Chapter 2" \l 1§

SOFTWARE UTILITIEStc "Software Utilities" \l 2§

This chapter describes the operation and installation of the following software utilities supplied on the Display Driver Diskettes:

n	CHIPSCPL

n	SETCOL

The CHIPSCPL utility programtc "The CHIPSCPL utility program" \l 3§

This utility program is designed to work with Microsoft Windows Version 3.1.

Installing the utility

CHIPSCPL.CPL is a Windows based utility to select resolutions and color depth.  It is a Control Panel Applet with its own icon that is automatically installed when installing CHIPS Windows 3.1 linear drivers.  The Control Panel icon is in the Main Windows group. To invoke the control panel applet, simply click on the icon.  The driver resolution and color depth take effect only after Windows is rebooted with the new driver.

How to use the utility

SIZE <ALT>+<S> allows you to select from the following resolutions:

640x480

800x600

1024x768

1280x1024

By selecting the resolution first, it will determine the allowable selections for color depth.

COLOR <ALT>+<O> allows you to select the number of colors from the following:

16 (4 bits per pixel)

256 (8 bpp)

32K (15bpp)

64K (16bpp)

16M (24bpp)

By selecting the color depth first, it will determine the allowable selections for resolution.

DISPLAY TYPE <ALT>+<D>  allows you select the display type from the following:

CRT only			<ALT>+<C>
	LCD (Flat Panel) only		<ALT>+<L>
	Both CRT and LCD (Flat Panel)	<ALT>+<B>

POWER SAVER <ALT>+<P>  allows you to save power by turning off either the back light or the panel after an inactive period of time.  This feature is available for the 65535 and 65540 only.

FONT <ALT>+<F>  allows you to select a large or small font.

VERSION <ALT>+<V>  displays version information about the current driver.

BIG CURSOR <ALT>+<G>  allows you to select a big cursor for better visibility on the Flat Panel.

The SETCOL utility programtc "The SETCOL utility program" \l 3§

This utility program is used to provide 132 text columns in popular text-based applications such as WordStar and WordPerfect.  If you do not intend to use 132 column text mode, do not install this utility.

Installing the utility

To install the utility, follow this procedure:

Step 1:	Determine where you want to store the program.  Locate a directory on your hard disk where you have other utility programs.  For your convenience, this directory should be specified in the PATH= statement in your AUTOEXEC.BAT file.

Step 2:	Place Display Driver Diskette #1 into drive A.  Make A the default drive by typing A:<ENTER>.  Run the SETUP program by typing SETUP <ENTER>.  Press any key to display a list of supported applications.  Use the arrow keys to select Utilities and press <ENTER>.  A list of utilities will be displayed.  Use the arrow keys to select the utilities desired and press <ENTER>.  Press <END> to begin the utility installation process.  A default drive and directory path will be displayed.  Use the backspace key to erase this default and type in the proper directory.  At this point you may be prompted to insert one of the other driver diskettes.  You also may be asked to create the target directory if it does not already exist.  After the files have been installed, press any key to return to the list of supported applications.  Press <ESC> followed by Y to exit to DOS.

How to use the utility

The SETCOL utility program allows you to specify the number of rows and columns on the screen.  You indicate these values to the SETCOL program by placing them after the name SETCOL on the command line.  The format for the command is:

SETCOL  COLUMNS, ROWS <ENTER>

Valid values for COLUMNS and ROWS are:

Columns		Rows

80 or 132		25 or 50

If you want to set 132 columns with 25 rows, give the following command:

SETCOL  132, 25 <ENTER>

Note 1:	To use this program with an application program such as WordStar or WordPerfect, the application may need to be configured for the specific screen size.  Please refer to other sections in this manual on how to configure your applications for this.

Note 2:	Certain monitors cannot display 50 character rows on the screen, due to hardware limitations in these monitors.  The following table specifies how many rows can be displayed on common monitors:

Monitor				Rows

IBM Monochrome			25

IBM Color Graphics Display		25

IBM Enhanced Graphics Display	25

IBM VGA Display (analog)		25 or 50

Multi-frequency Display		25 or 50