ADDRESS

| 0 | CONTROL |
| --- | --- |
| 1 | COMMAND |
| 2 | STATUS |
| 3 | INTERRUPT |
| 4 | ACCEPTANCE CODE |
| 5 | ACCEPTANCE MASK |
| 6 | BUS TIMING 0 |
| 7 | BUS TIMING 1 |
| 8 | OUTPUT CONTROL |
| 9 | TEST |

| 10 | IDENTIFIER. |
| --- | --- |
| 11 | RTR BIT. DATA LENGTH CODE |
| 12 | BYTE 1 |
| 13 | BYTE 2 |
| 14 | BYTE 3 |
| 15 | BYTE 4 |
| 16 | BYTE 5 |
| 17 | BYTE 6 |
| 18 | BYTE 7 |
| 19 | BYTE 8 |

| 20 | IDENTIFIER. |  | IDENTIFIER. |
| --- | --- | --- | --- |
| 21 | RTR BIT. DATA LENGTH CODE |  | RTR BIT. DATA LENGTH CODE |
| 22 | BYTE 1 |  | BYTE 1 |
| 23 | BYTE 2 |  | BYTE 2 |
| 24 | BYTE 3 |  | BYTE 3 |
| 25 | BYTE 4 |  | BYTE 4 |
| 26 | BYTE 5 |  | BYTE 5 |
| 27 | BYTE 6 |  | BYTE 6 |
| 28 | BYTE 7 |  | BYTE 7 |
| 29 | BYTE 8 |  | BYTE 8 |

| 31 | CLOCK DIVIDER |
| --- | --- |

Fig.3 PCX82C200 address allocation.

Table 1 Register map

| TITLE | ADDR | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Control Segment |  |  |  |  |  |  |  |  |  |
| Control Register | 0 | Test Mode | Sync | reserved | Overrun Interrupt Enable | Error Interrupt Enable | Transmit Interrupt Enable | Receive Interrupt Enable | Reset Request |
| Command Register | 1 | reserved | reserved | reserved | Goto Steep | Clear Overrun Status | Release Receive Buffer | Abort Trans-mission | Trans-mission Request |
| Status Register | 2 | Bus Status | Error Status | Transmit Status | Receive Status | Trans-mission Complete Status | Transmit Buffer Access | Data Overrun | Receive Buffer Stat |
| Interrupt Register | 3 | reserved | reserved | reserved | Wake-Up Interrupt | Overrun Interrupt | Error Interrupt | Transmit Interrupt | Receive Interrupt |
| Acceptance Code Register | 4 | AC.7 | AC.6 | AC.5 | AC.4 | AC.3 | AC.2 | AC.1 | AC.0 |
| Acceptance Mask Register | 5 | AM.7 | AM.6 | AM.5 | AM.4 | AM.3 | AM.2 | AM.1 | AM.0 |
| Bus Timing Register 0 | 6 | SJW.1 | SJW.0 | BRP.5 | BRP.4 | BRP.3 | BRP.2 | BRP.1 | BRP.0 |
| Bus Timing Register 1 | 7 | SAM | TSEG2.2 | TSEG2.1 | TSEG2.0 | TSEG1.3 | TSEG1.2 | TSEG1.1 | TSEG1.0 |
| Output Control Register | 8 | OCTP 1 | OCTN 1 | OCPOL 1 | OCTP 0 | OCTN 0 | OCPOL 0 | OCMODE 1 | OCMODE 0 |
| Test Register (note 1) | 9 | reserved | reserved | Map Internal Register | Connect RX Buffer 0 CPU | Connect TX Buffer CPU | Access Internal Bus | Normal RAM Connect | Float Output Driver |
| Transmit Butter |  |  |  |  |  |  |  |  |  |
| Identifier | 10 | ID.10 | ID.9 | ID.8 | ID.7 | ID.6 | ID.5 | ID.4 | ID.3 |
| RTR, Data Length Code | 11 | ID.2 | ID.1 | ID.0 | RTR | DLC.3 | DLC.2 | DLC.1 | DLC.0 |
| bytes 1-8 | 12-19 | Data | Data | Data | Data | Data | Data | Data | Data |
| Receive Buffer 0/1 |  |  |  |  |  |  |  |  |  |
| Identifier | 20 | ID.10 | ID.9 | ID.8 | ID.7 | ID.6 | ID.5 | ID.4 | ID.3 |
| RTR, Data Length Code | 21 | ID.2 | ID.1 | ID.0 | RTR | DLC.3 | DLC.2 | DLC.1 | DLC.0 |
| bytes 1-8 | 22-29 | Data | Data | Data | Data | Data | Data | Data | Data |
| Clock Divider | 31 | reserved | reserved | reserved | reserved | reserved | CD.2 | CD.1 | CD.0 |

Notes

1. 	The Test Register is used tor production testing only.

2. 	Register 30 is not implemented.

6.2.1 CONTROL REGISTER (CR)

The contents of the Control Register are used to change the behaviour of the PCX82C200. Control bits may be set or reset by the attached microcontroller which uses the Control Register as a read/write memory.

Table 2 Description of the Control Register bits

| CR | ADDRESS 0 |  |  |  |
| --- | --- | --- | --- | --- |
| BIT | SYMBOL | NAME | VALUE | FUNCTION |
| CR.7 | TM | Test Mode (note 1) | HIGH (enabled) LOW (disabled) | PCX82C200 enters Test Mode (normal operation impossible). Normal operating mode. |
| CR.6 | S | Sync (note 2) | HIGH (2 edges) LOW (1 edge) | Bus-line ransitions from recesslve-to-dominant and vice versa are used for resynchronization (see sections 7.2 and 8). Only transitions from recessive-to-dominant are used for resynchronization. |
| CR.5 |  |  |  | Reserved. |
| CR.4 | OIE | Overrun Interrupt Enable | HIGH (enabled) LOW (disabled) | If the Data Overrun bit is set (see section 6.2.3), the microcontroller receives an Overrun Interrupt signal. Microcontroller receives no Overrun Interrupt signal from the PCX82C200. |
| CR.3 | EIE | Error Interrupt Enable | HIGH (enabled) LOW (disabled) | If the Error or Bus Status change (see section 6.2.3), the microcontroller receives an Error Interrupt signal. Microcontroller receives no Error Interrupts signal. |
| CR.2 | TIE | Transmit Intrrupt Enable | HIGH (enabled) LOW (disabled) | When a message has been successfully transmitted or the transmit buffer is accessible again, (e.g. after an Abort Transmission command) the PCX82C200 transmits a Transmit Interrupt signal to the microcontroller. No transmission of the Transmit Interrupt signal by the PCX82C200 to the microcontroller. |
| CR.1 | RIE | Receive Interrupt Enable | HIGH (enabled) LOW (disabled) | When a message has been received without errors, the PCX82C200 transmits a Receive Interrupt signal to the microcontroller. No transmission of the Receive Interrupt signal by the PCX82C200 to the microcontroller. |
| CR.0 | RR | Reset Request (note 3) | HIGH (enabled) LOW (disabled) | Detection of a Reset Request resulis in the PCX82C200 aborting the current transmission or reception of a message and entering the reset state. On the HIGH-to-LOW transition of the Reset Request bit, the PCX82C200 retums to its normal operationg state. |

Notes

1. The Test Mode is Intended for factory testing and not for customer use.

2. The Sync bit should only be modified if the Reset Request bit is set HIGH (present). otherwise it is ignored. it is possible to set the Sync bit while the Reset Request bit is changed from HIGH to LOW.

3. During an extemal reset (RST = LOW) or when the Bus Status bit is set HIGH (Bus-Off), the IML forces the Reset Request HIGH (present). During an extemal reset the imcrocontroller cannot set the Reset Request bit LOW (absent). Therefore, after having set the Reset Request bit Low (absent), the microcontroller must check this bit to ensure the the extemal reset pin is not being held HIGH (present). After the Reset Request bit is set LOW to ensure that the extemal reset pin is not being held HIGH (present). After the Reset Request bit is set LOW (abset) the PCX82C200 will wait for:

	one occurrence of the Bus-Free signal (11 recessive bits, see section 8.9.6), if the preceding reset (reset Request = HIGH) was due to an extemal reset or a microcontroller initiated reset.

	128 occurrences of Bus-Free, If the preceding reset (Reset Request = HIGH) was due to a PCX82C200 Initiated Bus-Off, before re-entering the Bus-On mode (see section 8.9).

When Reset Request is set HIGH (present), for whatever reason, the control, command, status and interrupt bits are affected, see Table 3.

When Reset Request is set HIGH (present) the registers at addresses 4 to 8 are accessible but the TBF is not.

Table 3 Effects of setting the Reset Request bit HIGH (present)

| TYPE | BIT | FUNCTION | EFFECT |
| --- | --- | --- | --- |
| Control | CR.7 | Test Mode | LOW (disabled) |
| Command | CMR.4 CMR.3 CMR.2 CMR.1 CMR.0 | Goto Sleep Clear Overrun Status Release Receive Buffer Abort Transmission Transmission Request | LOW (wake-up) HIGH (clear) HIGH (released) LOW (absent) LOW (absent) |
| Status | SR.7 SR.6 SR.5 SR.4 SR.3 SR.2 SR.1 SR.0 | Bus Status Error Status Transmit Status Receive Status Transmission Complete Status Transmit Buffer Access Data Overrun Receive Buffer Status | LOW (Bus-On) (note 1) LOW (no error) (note 1) LOW (idle) LOW (idle) HIGH (complete) HIGH (released) LOW (absent) LOW (empty) |
| Interrupt | IR.3 IR.1 IR.0 | Overrun Interrupt Transmit Interrupt Receive Interrupt | LOW (reset) LOW (reset) LOW (reset) |

Note

Only after an extemal reset; see note 1 to Table 5 “Descrlption of the Status Register bits”.

6.2.2 COMMAND REGISTER (CMR)

A command bit initiates an action within the transfer layer of the PCX82C200. The Command Register appears to the microcontroller as a write only memory. If a read access is pertormed to this address the byte 11111111 (binary) is retumed.

Table 4 Description of the Command Register bits

| CMR | ADDRESS 1 |  |  |  |
| --- | --- | --- | --- | --- |
| BIT | SYMBOL | NAME | VALUE | FUNCTION |
| CMR.7 |  |  |  | Reserved. |
| CMR.6 |  |  |  | Reserved. |
| CMR.5 |  |  | § | Reserved. |
| CMR.4 | GTS | GoTo Sleep (note 1) | HIGH (sleep) LOW (wake up) | The PCX82C200 enters sleep mode, if the INT = HIGH (no interrupt signal from the PCX82C200 to the microcontroller pending or extemal source pending) and there is no bus activity. The PCX82C200 functions normally. |
| CMR.3 | COS | Clear Overrun Status (note 2) | HIGH (clear) LOW (no action) | The Data Overrun status bit is set to LOW (see section 6.2.3) No action. |
| CMR.2 | RRB | Release Receive Buffer (note 3) | HIGH (released) LOW (no action) | The receive buffer attached to the microcontroller is released. No action. |
| CMR.1 | AT | Abort Transmission (note 4) | HIGH (present) LOW (absent) | If not already in progress, a pending Transmission Request is cancelled. No action. |
| CMR.0 | TR | transmission Request (note 5) | HIGH (present) LOW (absent) | A message shall be transmitted. No action. |

Notes

1. The PCX82C200 will enter steep mode, if Goto Sleep is set HIGH (sleep), there is no bus activity and INT = HIGH (inactive). After sleep mode is set, the CLK OUT signal continues until at least 15 bit times have passed. The PCX82C200 will wake up when one of the three previously mentioned conditions is negated: after Goto Sleep is set LOW (wake up), there is bus activity or INT is driven LOW (active). On wake up, the oscillator is started and a Wake-Up Interrupt (see section 6.2.4) is generated. A PCX82C200 which is sleeping and then awakened by bus activity will not be able to receive this message until it detects a Bus-Free signal (section 8.9.6).

2. This command bit is used to acknowledge the Data Overrun condition signalled by the Data Overrun staus bit. It may be given or set at the same time as a Release Receive Buffer command bit.

3. After reading the contents of the Receive Buffer (RBF 0 of RBF 1) the microcontroller must release this buffer by setting the Release Receive Buffer bit HIGH (released). This may result in another message becoming immediately available.

4. The Abort Transmission bit is used when the microcontroller requires the suspension of the previously requested transmission, for example to transmit an urgent message. A transmission already in progress is not stopped. In order to determine if the original message had been transmitted successfully, of aborted, the Transmission Complete status bit should be checked after the Transmit Buffer Access bit has been set HIGH (released) or a Transmit Interrupt has been genurated (see section 6.2.4).

5. If the Transmission Request bit was set HIGH in a previous command, it cannot be cancelled by setting the Transmission Request bit LOW (absent). Cancellation of the requested transmission may be performed by setting the abort Transmission bit HIGH (present).

6.2.3 STATUS REGISTER (SR)

The contents of the Status Register reflect the status of the PCX82C200 bus controller. The Status Register appears to the microcontroller as a read only memory.

Table 5 Description of the Status Register bits

| CMR | ADDRESS 2 |  |  |  |
| --- | --- | --- | --- | --- |
| BIT | SYMBOL | NAME | VALUE | FUNCTION |
| SR.7 | BS | Bus Status (note 1) | HIGH (Bus-Off) LOW (Bus-On) | The PCX82C200 is not involved in bus activities. The PCX82C200 is involved in bus activities. |
| SR.6 | ES | Error Status | HIGH (error) LOW (ok) | At least one of the Error Counters (see section 8.10.3) has reached the microcontroller Warning Limit. Bothe Error Counters have not reached the warning Limit. |
| SR.5 | TS | Transmit Status (note 2) | HIGH (transmit) LOW (idle) | The PCX82C200 is transmitting a message. No message is transmitted. |
| SR.4 | RS | Receive Status (note 2) | HIGH (receive) LOW (idle) | The PCX82C200 is receiving a message. No message is received. |
| SR.3 | TCS | Transmission Complete Status (note 3) | HIGH (complete) LOW (incomplete) | Last requested transmission has been successfully completed. Previously requested transmission is not yet completed. |
| SR.2 | TBS | Transmit Buffer Access (note 3) | HIGH (released) LOW (locked) | The microcontroller may write a message into the TBF. The microcontroller cannot access the Transmit Buffer. A message is either waiting for transmission or is in the process of being transmitted. |
| SR.1 | DO | Data Overrun (note 4) | HIGH (overrun) LOW (absent) | This bit is set HIGH (Overrun), when both Receive Buffers are full and the first byte of another mesage should be stored. No data overrun has Occurred since the Clear Overrun command was given. |
| SR.0 | RBS | Receive Buffer Status (note 5) | HIGH (full) LOW (empty) | This bit is set when a new message is available. No message has become available since the last Release Receive Buffer command bit was set. |

Notes

1. When the Bus status bit is set HIGH (Bus-Off), the PCX82C200 will set the Reset Request bit HIGH (present). It will stay in this state until the microcontroller sets the Reset Request bit LOW (absent). Once this is completed the PCX82C200 will wait the minimum protocol-defined time (128) occurrences of the Bus-Free slgnal) before setting the Bus Status bit LOW (Bus-On), the Error Status bit LOW (ok) and resetting the Error Counters.

2. If both the Receivs Status and transmit Status bits are LOW (idle) the CAN-bus is idle.

3. If the microcontroller tries to write to the Transmit Buffer when the Teransmit Buffer Access bit is LOW (locked), the written bytes will not be accepted and will be lost without this being signalled. The Transmission Complete Status bit is set LOW (incomplete) whenever the Transmission Request bit is set HIGH (present). If an Abort transmission command is lssued, the Transmit Buffer will be released. IF the message, which was requested and then aborted, was not transmitted, the Transmission Complete Status bit will remain LOW.

4. If Data Overrun = HIGH (Overrun) is detected, the currently received message is dropped. A transmitted message, granted acceptance, is also stored in a Receive Buffer. This occurs because it is not known if the PCX82C200 will lose arbitration and so become a receiver of the message. If no Receive Buffer is available, Data Overrun is signalled.

5. If the command bit Release Receive Buffer is set HIGH (released) by the microcontroller,the Receive Buffer Status bit is set LOW (empty) by IML. When a new emssage is stored in any of the receive buffers, the Receive Buffer Status bit is set HIGH (full) again.

6.2.4 INTERRUPT REGISTER (IR)

The Interrupt Register allows the identification of an interrupt source. When one or more bits of this register are set, the INT pin is activated. All bits are rest by the PCX82C200 after this register is read by the microcontroller. This register appears to the microcontroller as a read only memory.

Table 6 Description of the Interrupt Register bits

| IR | ADDRESS 3 |  |  |  |
| --- | --- | --- | --- | --- |
| BIT | SYMBOL | NAME | VALUE | FUNCTION |
| IR.7 |  |  |  | Reserved. |
| IR.6 |  |  |  | Reserved. |
| IR.5 |  |  |  | Reserved. |
| IR.4 | WUI | Wake-Up Interrupt | HIGH (set) LOW (reset) | The Wake-Up Interrupt bit is set HIGH, when the sleep mode is left (see section 6.2.2). Wake-Up Interrupt bit is reset by a read access of Interrupt Register by the microcontroller. |
| IR.3 | CI | Overrun Interrupt (note 1) | HIGH (set) LOW (reset) | This bit is set HIGH, it both Receive Buffers contain a message and the first byte of another message should be stored (passed acceptance), and the Overrun Interrupt Enable is HIGH (enabled). Overrun Interrupt bit is reset by a read access of Interrupt Register by the microcontroller. |
| IR.2 | EI | Error Interrupt | HIGH (set) LOW (reset) | This bit is set on a change of either the Error Status or Bus Status bits (see section 6.2.3) if the Error Interrupt Enable is HIGH (enabled). The Error Interrupt bit is reset by a read access of the Interrupt Register by the microcontroller. |
| IR.1 | TI | Transmit Interrupt | HIGH (set) LOW (reset) | This bit is set on a change of the Transmit Buffer Access bit from Low to HIGH (released) and Transmit Interrupt Enable is HIGH (enabled). Transmit Interrupt bit will be reset after a read access of the Interrupt Register by the microcontroller. |
| IR.0 | RI | Receive Interrupt (note 2) | HIGH (set) LOW (reset) | This bit is set when a new message is available in the Receive Buffer and the Receive Interrupt Enable bit is HIGH (enabled). Receive Interrupt bit is automatically reset by a read access of Interrupt Register by the microcontroller. |

Notes

1. Overrun Interrupt bit (if enabled) and Data Overrun bit (see section 6.2.3) are set at the same time.

2. Receive Interrupt bit (if enabled) and Receive Buffer Status bit (see section 6.2.3) are set at the same time.

6.2.5 ACCEPTANCE CODE REGISTER (ACR)

The Acceptance Code Register is part of the acceptance filter of the PCX82C200. This register can be accessed (read/write), if the Reset Request bit is set HIGH (present). When a message is received which passes the acceptance test and if there is an empty Receive Buffer, then the respective Descrlptor and Data Field are sequentially stored in this empty buffer. In the case that there is no empty Receive Buffer, the Data Overrun bit is set HIGH (overrun), see sections 6.2.3 and 6.2.4. When the complete rnessage has been correctly received the following occurs:

	the Receive Buffer Status bit is set HIGH (full).

	if the Receive Interrupt Enable bit is set HIGH (enabled), the Receive Interrupt is set HIGH (set).

The Acceptance Code bits (AC.7-AC.0) and the eight most significant bits of the message's Identifier (ID.10-ID.3) must be equal to those bit positions which are marked relevant by the Acceptance Mask bits (AM.7-AM.0). If the following equation is satisfied, acceptance is given:

[ (ID.10 .. ID.3) = (AC.7 .. AC.0) ] or (AM.7 .. AM.0) = 1111 1111 binary

During transmission of a message which passes the acceptance test, the message is also written to its own Receive Buffer. If no Receiver Buffer is available, Data Overrun is signalled because it is not known at the start of a message whether the PCX82C200 will lose arbitration and so become a receiver of the message.

Table 7 Acceptance Code Register bits

| ACR | ADDRESS 4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| AC.7 | AC.6 | AC.5 | AC.4 | AC.3 | AC.2 | AC.1 | AC.0 |

6.2.6 ACCEPTANCE MASK REGISTER (AMR)

The Acceptance Mask Register is part of the acceptance filter of the PCX82C200. This register can be accessed (read/write) if the Reset Request bit is set HIGH (present). The Acceptance Mask Register qualifies which of the corresponding bits of the acceptance code are “relevant” for acceptance filtering.

Table 8 Acceptance Mask Register bits

| AMR | ADDRESS 5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| AM.7 | AM.6 | AM.5 | AM.4 | AM.3 | AM.2 | AM.1 | AM.0 |

Table 9 Desciption of the Acceptance Mask Register bits

| ACCEPTANCE MASK BIT | VALUE | COMMENTS |
| --- | --- | --- |
| AM.7 to AM.0 | HIGH (don't care) LOW (relevant) | This bit position is “don't care” for the acceptance of a message. This bit position is “relevant” for acceptance filtering. |

6.2.7 BUS TIMING REGISTER 0 (BTR0)

The contents of Bus Timing Register 0 defines the values of Baud Rate Prescaler (BRP) and the Synchronization Jump Width (SJW). This register can be accessed (read/write) if the Reset Request bit is set HIGH (present).

Table 10 Bus Timing Register 0 bits

| BTR0 | ADDRESS 6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| SJW.1 | SJW.0 | BRP.5 | BRP.4 | BRP.3 | BRP.2 | BRP.1 | BRP. |

Baud Rate Prescaler (BRP)

The period of the system clock tSCL rogrammable and determines the individual bit timing. The system clock is calculated using the following equation:

tSCL = 2tclk (32brp.5 + 16BRP.4  + 8BRP.3 + 4BRP.2 + BRP.0 +1)

tCLK  = time period of the PCX82C200 oscillator.

Synchronizatuion Jump Width (SJW)

To compensate for phase shilts between clock oscillators of different bus controllers, any bus controller must resynchronize on any relevant signal edge of the current transmission. The synchronization jump width delines the maximum number of clock cycles a bit period may be shortened or lengthened by one resynchronization:

tSJW  = tSCL (2SJW.1 + SJW.0 + 1)

For further information on bus timing see sections 6.2.8 and 7.

6.2.8 BUS TIMING REGISTER 1 (BTR1)

The contents of Bus Timing Register 1 delines the length of the bit period, the location of the sample point and the number of samples to be taken at each sample point. This register may be accessed (read/write) it the Reset Request bit is set HIGH (present).

Table 11 Bus Timing Register 1 bits

| BTR1 | ADDRESS 7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| SAM | TSEG2.2 | TSEG2.1 | TSEG2.0 | TSEG1.3 | TSEG1.2 | TSEG1.1 | TSEG1.0 |

Table 12 Selection of sampling

| BIT | VALUE | COMMENTS |
| --- | --- | --- |
| SAM | HIGH (3 samples) LOW (1 sample) | Three samples are taken. The bus is sampled once. |

SAM-LOW (logic 0) is recommended for high speed buses (SAE class C). while SAM = HIGH (logic 1) is recommended for slow/medium speed buses (class A and B) where filtering of spikes on the bus-line is beneficial (see section 7.1.6).

Time Segment 1 (TSEG1) and Time Segment 2 (TSEG2)

TSEG1 and TSEG2 determine the number of clock cycles per bit penod and the location of the sample point:

tTSEG1 = tSCL (8TSEG1.3 + 4TSEG1.2 + 2TSEG1.1 + TSEG2.0 + 1)

tTSEG2  = tSCL (4TSEG2.2 + 2TSEG2.1 + TSEG2.0 + 1)

For further information on bus timing see sections 6.2.7 and 7.

6.2.9 OUTPUT CONTROL REGISTER  (OCR)

The Output Control Register allows, under software control, the set-up of different output driver configurations. This register may be accessed (read/write) if the Reset Request bit is set HIGH (present).

Table 13 Output Control Register 1 bits

| OCR | ADDRESS 8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| OCTP1 | OCTN1 | OCPOL1 | OCTP0 | OCTN0 | OCPOL0 | OCMODE1 | OCMODE0 |

If the PCX82C200 is in the sleep mode (Goto Sleep = HIGH) a recessive level is output on the TX0 and TX1 pins. If the PCX82C200 is in the reset state (Reset Request = HIGH) the output drivers are floating.

Normal Output Mode

In Normal Output Mode the bit sequence (TXD) is sent via TX0 and TX1. The voltage levels on the output driver pins TX1 and TX0 depend on both the driver characteristic programmed by OCTPx, OCTNx (float, pull-up , pull-down, push-pull and the output polarity programmed by OCPOLx (see Fig.4).

µ §

Clock Output Mode

For the TX0 pin this is the same as in Normal Output Mode. However, the data stream to TX1 is replaced by the transmit clock (TXCLK). THe rising edge of the transmit clock (non inverted) marks the beginning of a bit period. The clock putse width is tSCL.

Bi-phase Output Mode

In contrast to Normal Output Mode the bit representation is time variant and toggled. If the bus controllers are galvanically decoupled from the bus-line by a transformer, the bit stream is not allowed to contain a DC component. This is achieved by the following scheme. During recessive bits all outputs are deactivated (3-state). Dominant bits are sent altematingly on TX0 and TX1, i.e. the first dominant bit is sent on TX0, the second is sent on TX1, and the third one is sent on TX0 again, etc.

Test Output Mode

For the TX0 pin this is the same as in Normal Output Mode. To measure the delay time of the transmitter and receiver this mode connects the output of the input comparator (COMP OUT) with the input of the output driver TX1. This mode is used for production testing only.

The following two tables, Table 14 and Table 15, show the relationship between the bits of the Output Control Register and the two serial output pins TX0 and TX1 of the PCX82C00, connected to the serial bus (see Fig.1).

Table 14 Description of the Output Mode bits

| OCMODE1 | OCMODE0 | COMMENTS |
| --- | --- | --- |
| 1 | 0 | Normal Output Mode; TX0, TX1: bit sequence (TXD; note1). |
| 1 | 1 | Clock Output Mode; TX0: bit sequence, TX1: bus clock (TXCLK). |
| 0 | 0 | Bi-phase Output Mode. |
| 0 | 1 | Test Output MOde; TX0: bit sequence, TX1: COMP OUT. |

Note

TXD is the data bit to be transmitted.

Table 15 Output pin set-up

| DRIVE | OCTPx | OCTNx | OCPOLx | TXD | TPx (note 1) | TNx (note 2) | TXx (note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Float | 0 0 0 0 | 0 0 0 0 | 0 0 1 1 | 0 1 0 1 | OFF OFF OFF OFF | OFF OFF OFF OFF | float float float float |
| Pull-down | 0 0 0 0 | 1 1 1 1 | 0 0 1 1 | 0 1 0 1 | OFF OFF OFF OFF | ON OFF OFF ON | LOW float float LOW |
| Pull-up | 1 1 1 1 | 0 0 0 0 | 0 0 1 1 | 0 1 0 1 | OFF ON ON OFF | OFF OFF OFF OFF | float HIGH HIGH float |
| Push/Pull | 1 1 1 1 | 1 1 1 1 | 0 0 1 1 | 0 1 0 1 | OFF ON ON OFF | ON OFF OFF ON | LOW HIGH HIGH LOW |

Notes

1. TPx is the on-chip output transistor x, connected to VDD; x = 0 or 1.

2. TNx is the on-chip output transistor x, connected to VSS; x = 0 or 1.

3. TXx is serial output level on pin TX0 or TX1. It is required that the output level on the CAN-bus is dominant with TXD = 0 and recessive with TXD = 1 (see section 8.1.1).

6.2.10 TEST REGISTER  (TR)

The Test Register is used for production testing only.

6.3 Transmit Buffer layout

The global layout of the Transmit Buffer is shown in Fig.3. This buffer serves to store a message from the microcontroller to be transmitted by the PCX82C200 . It is subdivided into Descriptor and Data Field. The Transmit Buffer can be written to and read from by the microcontroller (see note 3 to Table 2).

6.3.1 DESCRIPTOR

Table 16 Descriptor Byte 1 (DSCR1)

| DSCR1 | ADDRESS 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| ID.10 | ID.9 | ID.8 | ID.7 | ID.6 | ID.5 | ID.4 | ID.3 |

Table 17 Descriptor Byte 2 (DSCR2)

| DSCR2 | ADDRESS 11 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| ID.2 | ID.1 | ID.0 | RTR | DLC.3 | DLC.2 | DLC.1 | DLC.0 |

Identifier (ID)

The Identifier consists of 11 bits (ID.10 to ID.0). ID.10  is the most significant bit, which is transmitted first on the bus during the arbitration process. The Identiffer acts as the message's name, used in a receiver for acceptance filtering and also determines the bus access priority during the arbitration process. The lower the binary value of the ldentifier the higher the priority. This is due to the larger number of leading dominant bits during arbitration (see section 8.7 “Bus organization”).

Remote Transmission Request bit (RTR)

Table 18 Description of the RTR bit

| BIT | VALUE | COMMENTS |
| --- | --- | --- |
| RTR | HIGH (remote) LOW (data) | Remote Frame will be transmitted by the PCX82C200. Data Frame will be transmitted by the PCX82C200. |

Data Length Code (DLC)

The number of bytes (Data Byte Count) in the Date Field of a message is coded by the Data Length Code. At the start of a Remote Frame transmission the Data Length Code is not considered due to the RTR bit being HIGH (remote). This forces the number of transmitted/received data bytes to be 0. Nevertheless, the Data Length Code must be specified correctly to avoid bus errors, if two Can-controllers start a Remote Frame transmission simultaneously.

The range of the Data Byte Count is 0 to 8 bytes and coded as follows:

Data Byte Count = 8DLC.3 + 4DLC.2 + 2DLC.1 + DLC.0

For reasons of compatibllity no Data Byte Counts other than 0 to 8 should be used.

6.3.2 DATA FIELD

The number of transferred data bytes is determined by the Data Length Code. The first bit transmitted is the most significant bit of data byte 1 at address 12.

6.4 Receive Buffer Layout

The layout of the Receive Buffer and the individual bytes correspond to the definitions given for the Transmit Buffer layout, except that the addresses start at 20 instead of 10 (see Fig.3).

6.5 Clock Divider Register  (CDR)

The Clock Divider Register controls the CLK OUT frequency for the microcontroller MIC-263 no use this pin (see Fig.1). It can be written to or read by the microcontroller. The default state of the register is divide by 12 for Motorola mode and divide by 2 for intel mode. Values from 0 to 7 may be written into this register and will result in the CLK OUT frequencies shown in table 20.

Table 19 Clock Divider Register Bits

| CDR | ADDRESS 31 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| - | - | - | - | - | CD.2 | CD.1 | CD.0 |

Table 20 CLK OUT frequency selection

| CD.2 | CD.1 | CD.0 | CLK OUT FREQUENCY |
| --- | --- | --- | --- |
| 0 0 1 0 1 1 1 1 | 0 0 1 1 0 0 1 1 | 0 1 0 1 0 1 0 1 | µ § µ § µ § µ § µ § µ § µ § µ § |

Note

tCLK is the frequency of the oscillator.

7 BUS TIMING/SYNCHRONIZATION

The Bus Timing Logic (BIL) monitors the serial bus-line via the on-chip input comparator and pertoms the following functions (see sections 5):

	monitors the serial bus-line level

	adjusts the sample point, within a bit period (programmable)

	samples the bus-line level using majority logic (programmable, 1 or 3 samples)

	synchronization to the bit stream:

	hard synchronization at the start of a message

	resynchronization during transfer of a message

The configuration of the BTL is performed during the initlalization of the PCX82C200. The BTL uses the following three registers:

	Control register (Sync)

	Bus Timing Register 0

	Bus Timing Register 1

7.1 Bit timing

A bit period is built up from a number of system clock cycles (tSCL), see section 6.2.7. One bit period is the result of the addition of the programmable segments TSEG1 and TSEG2 and the general segment SYNCSEG (see sections 6.2.7 to 6.2.8).

7.1.1 SYNCHRONIZATION SEGMENT (SYNCSEG)

The incoming edge of a bit is expected during this state; this state  corresponds to one  system clock cycle (1 x tSCL).

7.1.2 TIME SEGMENT 1 (TSEG1)

This segment determines the location of the sampling point within a bit period, which is at the end of TSEG1. TSEG1 is programmable from 1 to 16 system clock cycles (see section 6.2.8).

The correct location of the sample point is essential for the correct functioning of a transmission. The following points must be taken into conideration:

	a Start-Of-Frame (see section 8.2.1) causes all PCX82C200's to perform a 'hard synchronization' (see section 7.2.1) on the first recessive-to-dominant edge. During arbitration, however, several PCX82C200's may simultaneously transmit. Therefore it may require twice the sum of bus-line, input comparator and the output driver delay times until the bus is stable. This is the propagation delay time.

	to avoid sampling at an incorrect position, it is necessary to include an additional synchronization buffer on both sides of the sample point. The main reasons for incorrect sampling are:

	incorrect synchronization due to spikes on the bus-line

	slight variations in the oscillator frequency of each PCX82C200 in the network, which results in a phase error.

Time Segment 1 consists of the segment for compensation of propagation delays and the synchronization buffer directly before the sample point (see Fig.5).

7.1.3 TIME SEGMENT 2 (TSEG2)

This time segment provides:

	additional time at the sample point for calculation of the subsequent bit levels (e.g. arbitration)

	synchronization buffer segment directly after the sample point (see section 7.1.2).

TSEG2 is programmable from 1 to 8 system clock cycles (see section 6.2.8).

µ §

7.1.4 SYNCHRONIZTION JUMP WIDTH (SJW)

SJW defines the maximum number of clock cycles (tSCL) a bit period may be reduced or increased by one resynchronization. SJW is programmable from 1 to 4 system clock cycles (see section 6.2.7).

7.1.5 PROPAGATION DELAY TIME

The propagation delay time (tprop) is calculated by summing the maximum propagation delay times of the physical bus, the input comparator and the output driver. The resulting sum is multiplied by 2 and then rounded up to the nearest multiple of tSCL.

tprop = 2 x (physical bus delay + input comparator delay + output driver delay)

7.1.6 BIT TIMING RESTRICTIONS

Restrictions on the configuration of the bit timine are based on internal processing. The restrictions are:

µ §

The three sample mode (SAM = 1) has the effect of introducing a delay of one system clock cycle on the bus-line. This must be taken into account for the correct calculation of  TSEG1 and TSEG2:

µ §

7.2 Synchronization

Synchronization is performed by a state machine which compares the incoming edge with its actual bit timing and adapts the bit timing by hard synchronization or resynchronization.

7.2.1 HARD SYNCHRONIZATION

This type of synchroization occurs only at the beginning of a message. the PCX82C200 synchronizes on the first incoming recessive-to-dominant edge of a message (being the leading edge of a message's Start-Of-Frame bit; see section 7-1).

7.2.2 RESYNCHRONIZATION

Resynchronization occurs during the transmission of a message's bit stream to compensate for:

	variations in individual PCX82C200 oscillator frequencies.

	changes introduced by switching from one transmitter to another (e.g. during arbitration).

As a result of resynchronization either tTSEG1 may be increased by up to a maximum of tSJW or tTSEG2 may be decreased by up to a maximum of tSJW:

| µ § | ((TESG1 + 1) + (SJW + 1)) ((TESG2 + 1)  (SJW + 1)) |
| --- | --- |

Note: TSEG1, TSEG2 and SJW are the programmed numerical values.

The phase error (e) of an edge is given by the position of the edge relative to SYNCSEG, measured in system clock cycles  (tSCL). The value of the phase error is defined as:

	e = 0, if the edge occurs within SYNCSEG

	e > 0, if the edge occurs within TSEG1

	e < 0, if the edge occurs within TSEG2

The effect of resynchronization is:

	the same as that of a hard synchronization, if the magnitude of the phase error (e) is less or equal to the programmed value of tSJW (see section 6.2.7).

	to increase a bit period by the amount of tSJW, if the phase error is positive and the magnitude of the phase error is larger than tSJW.

	to decrease a bit period by the amount of tSJW, if the phase error is negative and the magnitude of the phase error is larger than tSJW.

7.2.3 SYNCHRONIZATION RULES

The synchronization rules are as follows:

	only one synchronization within one bit time is used

	an edge is used for synchronization only if the value detected at the previous sample point differs from the bus value immediately after the edge

	hard synchronization is performed whenever there is a recessive-to-dominant edge during Bus-ldle (see section 7).

	all other edges (recessive-to-dominant and optionally dominant-to-recessive edges if the Sync bit is set HGH; see section 6.2.1) which are candidates for resynchronization will be used with the following exception:

	a transmitting PCX82C200 will not perform a resynchronization as a result of a recessive-to-dominant edge with positive phase error, if only these edges are used for resynchronization. This ensures that the delay times of the output driver and input comparator do not cause a permanent increase in the bit time.

8 COMMUNICATION PROTOCOL

8.1 Frame types

The PCX82C200 bus controller supports the four different CAN-protocol frame types for communication:

	Data Frame, to transfer data

	Remote Frame, request for data

	Error Frame, globally signal a (locally) detected error condition

	Overload Frame, to extend delay time of subsequent frames (an Overload Frame is not initiated by the PCX82C200).

8.1.1 BIT REPRESENTATION

There are two logical bit representations used in the CAN-protocol:

	a recessive bit on the bus-line appears only if all connected PCX82C200's send a recessive bit at that moment

	dominant bits always overwrite recessive bits i.e. the resulting bit level on the bus-line is dominant.

8.2 Date Frame

A Data Frame carries data frome a transmitting PCX82C200 to one or more receiving PCX82C200's A Data Frame is composed of seven different bit-fields:

	Start-O-Frame

	Arbitration Field

	Control Fiedl

	Data Field (may have a length of zero)

	CRC Field

	Acknowledge Field

	End-Of-Frame

µ §