***************************************************************

*             How to load PCM-3718 Device Driver into         *

*             BASICA and QuickBASIC 4.0, 4.5                  *

***************************************************************

To provide additional programming language support, the PCM-3718

offers a software driver that is for QuickBASIC 4.0 and 4.5

programming.  The file name of this driver is "3718QB.QLB" and it

is available in your PCM-3718 software diskette.

The programming syntax of the QuickBASIC driver is exactly the

same as the BASICA driver described in the manual.  However, there

are minor changes regarding the loading and initialization of the

QuickBASIC driver.

Loading the Driver

In BASICA and GWBASIC 2.02, the routine to load PCM-3718 driver is

like:

110 'LOAD 3718BAS.BIN DRIVER TO BASIC AREA

120 CLEAR ,57344!

130 DEF SEG=0

140 SG=256*PEEK(&H511)+PEEK(&H510)

150 SG=SG+57344!/16

160 DEF SEG=SG

170 BLOAD "3718BAS.BIN",0

180 'END OF DRIVER LOADING

In GWBASIC 3.22, the routine to load PCM-3718 driver is

like:

110 'LOAD 3718BAS.BIN DRIVER TO AN OUTSIDE AREA

120 DEF SEG=&H6000

130 BLOAD "3718BAS.BIN",0

140 'END OF DRIVER LOADING

In QuickBASIC, since the PCM-3718 driver is treated as a library,

it is not necessary to load PCM-3718 driver to the work area as it

is in BASICA.  Please eliminate the above coding from program

originally written for BASICA before using it for QuickBASIC. To

make the PCM-3718 driver ready for use, users must key in the

following command when entering QuickBASIC.

QB filename /L 3718QB.QLB

If format error occurred for 3718QB.QLB when you run QB, you may make

your own 3718QB.QLB by following steps:

Under Quick BASIC 4.5:

LINK 3718QB.OBJ /Q,,,BQLB45.LIB;

Under Micorsoft BASIC 7.0/7.1:

LINK 3718QB.OBJ /Q,,,QBXQLB.LIB;

Initializing the Driver

In BASICA and GWBASIC, the routine to initialize PCM-3718 driver is

like:

100 PORT% = &H300

110 PARAM%(0) = 0     'Board number

120 PARAM%(1) = PORT%

**   140 PCM3718 = 0      'DRIVER OFFSET

160 FUNC% = 3

170 CALL PCM3718(FUNC%, PARAM%(0))

180 IF ERR% <>0 THEN PRINT "INSTALLATION FAILED !":STOP

In QuickBASIC, the PCM-3718 driver is treated as a library, users

do not have to specify the PCM-3718 driver offset. Please delete

this following line from the initialization routine.

140 PCM3718 = 0

If the user wants to run the example programs offered with PCM-3718

in QuickBASIC, the routine section to load "3718BAS.BIN" and

the statement "PCM3718=0" must be deleted. Besides, a statement

such as:

DIM DTA%(N), CH%(N)

must be changed to a statement as

DIM DTA%(500), CH%(500)

The size of these arrays can be smaller or larger and it must be

a constant otherwise QuickBASIC will show "Overflow" error.