PCM-3864 Utility Disk (Ver 3.01)  Date : 8/14/1998

-----------------------------------------------------------------------------

The PCM-3864 is supplied with a software utility disk. This disk contains the

necessary file for setting up the VGA display.

Directories and files on the disk are as follows:

ROOT

�  README.DOC

�  AMIFLASH.COM

�

��� 3864_EL.ROM

3864_MON.ROM

3864_ST5.ROM

3864_TFT.ROM

3864_SYS.ROM

3864_CRT.ROM

AMIFLASH.COM

------------

This program allows you to write the VGA display driver files to the BIOS

Flash ROM. The VGA files all come ready formatted for the PCM-3864 with .BIN

extensions. See README.DOC. These files support various CRT and flat-panel

displays. They are custom written and can be made available upon request.

3864_SYS.ROM

------------

This binary file contains the system BIOS.

3864_CRT.ROM

------------

This binary file supports CRT display only.

3864_TFT.ROM (Default)

------------

Supports 640x480 color TFT (Sharp LQ9D011, Toshiba LTM09C015A/016)

3864_ST5.ROM (512KB VRAM)

------------

supports 640x480 color STN DD 8/16-bit displays (Sharp LM64C142)

NOTE: This BIOS does not support simultaneous CRT

and flat panel diaplay with 512 KB VRAM.

3864_MON.ROM

------------

Supports 640x480 dual scan monochrome displays (Sharp LM64183)

3864_EL.ROM

------------

Supports 640x480 EL displays (PLANAR EL640.480-A Series)