PCM-4823 Utility Disk V1.21              2/02/1999

-----------------------------------------------------------------------------

The PCM-4823 is supplied with a software utility disk. This disk contains the

necessary file for setting up the VGA display WDT and LAN controller.

Directories and files on the disk are as follows:

ROOT

*

README.DOC

4823-SYS.BIN

4823-STN.BIN

4823-EL.BIN

4823-MON.BIN

4823-CRT.BIN

4823-TFT.BIN

AWDFLASH.EXE

CBROM.EXE

AWDFLASH.EXE

DISABLE.ASM

DISABLE.EXE

ENABLE.ASM

ENABLE.EXE

REFRESH.COM

REFRESH.BAT

TEST1.BAT

TEST2.BAT

------------

This program allows you to write the VGA display driver files to the BIOS

Flash ROM. The VGA files all come ready formatted for the PCM-4823 with .BIN

extensions. These files support various CRT and flat-panel displays. They

are custom written and can be made available upon request.

4823-SYS.BIN

------------

This binary file contains the system BIOS.

4823-TFT.BIN

------------

Supports 640x480 color 18/24-BIT TFT (Sharp LQ9D011, Toshiba LTM10C042)

4823-STN.BIN

------------

Supports 640x480 color STN DD 8/16-bit displays (Sharp LM64C142)

4823-MON.BIN

------------

Supports 640x480 dual scan monochrome displays (Sharp LM64183)

4823-EL.BIN

------------

Supports 640x480 EL displays (PLANAR EL640.480-A Series)

4823-CRT.BIN

------------

This binary file supports CRT display only.

CBROM.EXE

------------

This program allows you to combine your own VGA BIOS with system BIOS.

If you have your own VGA BIOS ( which supports special flat panel ) for

PCM-4823, please update your BIOS refer to the following example:

COPY 4823-SYS.BIN 4823-NEW.BIN

CBROM 4823-NEW.BIN /VGA NEW-VGA.DAT

AWDFLASH 4823-NEW.BIN

NEW-VGA.DAT : Your own VGA BIOS file (which supports special LCD) for PCM-4823.

4823-NEW.BIN: The NEW BIOS supports your own special LCD for PCM-4823.

DISABLE.ASM:		Source code of DISABLE.EXE(WDT)

DISABLE.EXE:		Disable WDT function(WDT)

ENABLE.ASM:		    Source code of ENABLE.EXE(WDT)

ENABLE.EXE:		    Enable WDT function(WDT)

REFRESH.COM:		Refresh/ Initiate  time interval of WDT(WDT)

REFRESH.BAT:		Test batch file(WDT)

TEST1.BAT:		    Test batch file(WDT)

TEST2.BAT:		    Test batch file(WDT)

-------------------------------------------------------------------------------

WDT Test Example:

(1) TEST1.BAT: This batch file initiates time interval of WDT, then enables WDT function. After executing this batch file, if you don’t press any key within 1.6 seconds, the system will be reset.

(2) TEST2.BAT: This batch file initiates time interval of WDT and enables WDT function. If you press any key within 1.6 seconds, the WDT function will be disabled, otherwise WDT makes system reboot.

-------------------------------------------------------------------------------