PCM-4825 Utility Disk (Rev.1.20)  Date :11/27/1997

-----------------------------------------------------------------------------

The PCM-4825 is supplied with a software utility disk. This disk contains the

necessary file for setting up the VGA display.

Directories and files on the disk are as follows:

ROOT

README.DOC

AWDFLASH.EXE

4825_SYS.BIN

4825_CRT.BIN

4825_TFT.BIN

4825_STN.BIN

4825_MON.BIN

4825_EL.BIN

CBROM.EXE

SM.COM

CT.COM

FP.COM

ON.COM

OFF.COM

AWDFLASH.EXE

------------

This program allows you to write the VGA display driver files to the BIOS

Flash ROM. The VGA files all come ready formatted for the PCM-4825 with .BIN

extensions. They are custom written and can be made available upon request.

4825_SYS.BIN

------------

This binary file contains the system BIOS

4825_CRT.BIN

------------

Supports CRT only

4825_TFT.BIN

------------

Supports 640X480 color TFT

(Toshiba LTM10C042)

4825_STN.BIN

------------

supports 640X480 color STN DD 8/16-bit displays

(Sharp LM64C142)

4825_MON.BIN

------------

Supports 640X480 dual scan monochrome displays

(Sharp LM64P83/64P89)

4825_EL.BIN

-----------

Supports 640X480 EL displays

(PLANAR EL640X480-A series)

CBROM.EXE

----------

This program allows you to combine your own VGA BIOS with

system BIOS(4825_SYS.BIN)

SM.COM

------

Enables both displays at the same time

CT.COM

------

Enables CRT display only

FP.COM

------

Enables panel display only

ON.COM

------

Switches to normal display mode

OFF.COM

-------

Switches to sleep mode