PCM-4862 Utility Disk (V2.40)  Date : 12/16/1998

-----------------------------------------------------------------------------

The PCM-4862 is supplied with a software utility disk. This disk contains the

necessary file for setting up the VGA display and Ethernet controller.

Directories and files on the disk are as follows:

ROOT

�   README.DOC

�

����SSD

�       COOKROM.EXE

�

����BIOS

4862-EL.BIN

4862-MON.BIN

4862-ST5.BIN

4862-STN.BIN

4862-TFT.BIN

AWDFLASH.EXE

CBROM.EXE

4862-SYS.BIN

4862-CRT.BIN

AWDFLASH.EXE

------------

This program allows you to write the VGA display driver files to the BIOS

Flash ROM. The VGA files all come ready formatted for the PCM-4862 with .BIN

extensions. See README.DOC. These files support various CRT and flat-panel

displays. They are custom written and can be made available upon request.

4862-SYS.BIN

------------

This binary file contains the system BIOS.

4862-CRT.BIN

------------

This binary file supports CRT display only.

4862-TFT.BIN (Default)

------------

Supports 24-bit 640x480 color TFT (Sharp LQ9D011, Toshiba LTM10C042)

4862-STN.BIN (1MB VRAM)

------------

supports 640x480 color STN DD 8/16-bit displays (Sharp LM64C142)

4862-ST5.BIN (512KB VRAM)

------------

supports 640x480 color STN DD 8/16-bit displays (Sharp LM64C142)

NOTE: This BIOS does not support simultaneous CRT

and flat panel diaplay with 512 KB VRAM.

4862-MON.BIN

------------

Supports 640x480 dual scan monochrome displays (Sharp LM64183)

4862-EL.BIN

------------

Supports 640x480 EL displays (PLANAR EL640.480-A Series)

CBROM.EXE

------------

This program allows you to combine your own VGA BIOS with system BIOS.

If you have your own VGA BIOS ( which supports special flat panel ) for

PCM-4862 , please update your BIOS refer to the following example:

COPY 4862-SYS.BIN 4862-NEW.BIN

CBROM 4862-NEW.BIN /VGA NEW-VGA.DAT

AWDFLASH 4862-NEW.BIN

NEW-VGA.DAT : Your own VGA BIOS file (which supports special LCD) for PCM-4862.

4862-NEW.BIN: The NEW BIOS supports your own special LCD for PCM-4862.

-------------------------------------------------------------------------------