PCM-5820 Utility Disk (V1.10)  Date : 2/4/1999

-----------------------------------------------------------------------------

The PCM-5820 is supplied with a software utility disk. This disk contains the

necessary file for setting up the LCD control parameters.

ROOT

�   README.DOC

�

�

����BIOS

5820V110.BIN

AWDFLASH.EXE

CBROM.EXE

LCD.BIN

PANEL.EXE

AWDFLASH.EXE

------------

This program allows you to write the new BIOS file to the BIOS Flash ROM.

5820V110.BIN

------------

This binary file contains the original system BIOS.

CBROM.EXE

------------

This program allows you to combine your own LCD parameter file with

the original system BIOS.

If you have your own LCD file ( which supports special flat panel ) for

PCM-5820 , please update your BIOS refer to the following example:

COPY 5820V110.BIN 5820-NEW.BIN

CBROM 5820-NEW.BIN /other 1800:0 LCD.BIN

AWDFLASH 5820-NEW.BIN

PANEL.EXE

------------

A windows application program. It can be used to acquire LCD control parameters.

LCD.BIN

------------

The default LCD control parameter file.

This file contains three types of LCD control parameter in three groups.

Each group consists of 30 bytes. The first group is for 640X480 LCD panel.

the second group is for 800X600 LCD panel, and the third group is for 1024X768

LCD panel. The format of each group is describe in follows:

Group 1:     (INDEX,VALUE)

BYTE 1,2        51H,xxH         ;Type

BYTE 3,4        52h,xxH         ;Resolution

BYTE 5,6        53H,xxH         ;Control

BYTE 7,8        54H,xxH         ;Dither

BYTE 9,10       55H,xxH         ;CLOCK

BYTE 11,12      56H,xxH         ;HTOTAL

BYTE 13,14      57H,xxH         ;VTOTAL

BYTE 15,16      58H,xxH         ;LCD HSYNC START

BYTE 17,18      59H,xxH         ;LCD HSYNC END

BYTE 19,20      5AH,xxH         ;LCD VSYNC START

BYTE 21,22      5BH,xxH         ;LCD VSYNC END

BYTE 23,24      5CH,xxH         ;CRT HSYNC START

BYTE 25,26      5DH,xxH         ;CRT HSYNC END

BYTE 27,28      5EH,xxH         ;CRT VSYNC START

BYTE 29,30      5FH,xxH         ;CRT VSYNC END

------------------------------------------------------------------------------