--------------------   The RTL8029 Boot ROM  ----------------------

OutLines:

A. How to create the RTL8029 Novell RPL Boot ROM Code.

B. How to install the Novell 3.X 4.X RPL server.

C. How to install the Microsoft Windows NTAS 3.5 , 3.51 RPL server.

D. How to install the Microsoft LAN Manager 2.1 Remoteboot server.

E. How to install the IBM OS/2 LAN Server 4.0 Remoteboot server.

A. How to create the RTL8029 Novell RPL Boot ROM code

1. Using Borland's TLINK.EXE to create a file called RBOOT.ROM. For example.

TLINK BSM.OBJ BSMETHER.OBJ PCIODI.OBJ, RBOOT.ROM /t

Note:

TLINK.EXE must be version 6.00 or higher.

DPMI16BI.OVL and RTM.EXE must also be in the same directory with TLINK.EXE.

2. Run the ROMSUM utility to adjust the size of RBOOT.ROM. For example,

ROMSUM   16

3. Test the Boot ROM's functionality by using TESTROM.COM.

4. Burn the Boot ROM into a PROM.

Note:

The Novell Boot ROM Developer's kit (RPL_KIT) provides the following files:

BSM.OBJ

BSMETHER.OBJ

ROMSUM.COM

TESTROM.COM

The Realtek Semiconductor provides the object module PCIODI.OBJ.

B. How to install the Novell RPL Server

1. You must create a remote boot diskette. The files on this boot diskette,

for example, are the following:

COMMAND  COM

IO       SYS   attrib -s -h io.sys

MSDOS    SYS   attrib -s -h msdos.sys

LSL      COM

PCIODI   COM

IPXODI   COM

NETX     EXE

AUTOEXEC BAT

CONFIG   SYS

The AUTOEXEC.BAT file contain these lines :

PATH H:\LOGIN;

SET COMSPEC=H:\LOGIN\COMMAND.COM

DOSKEY

LSL

PCIODI

IPXODI

NETX

The CONFIG.SYS file  contain these lines :

DEVICE=A:\HIMEM.SYS

STACKS=9,256

DOS=HIGH,UMB

FILES=30

BUFFERS=30

LASTDRIVE=G

FCBS=16,8

2. Boot a workstation with at least one floppy disk drive, and log in to

the server as SUPERVISOR.

3. Change directory to SYS:SYSTEM

4. With the DOS boot diskette in driver A: execute the DOSGEN utility. This

creates a DOS boot image file named NET$DOS.SYS.

5. Copy NET$DOS.SYS to the SYS:LOGIN directory.

6. Remoteboot different DOS image files by RPL server

Create BOOTCONF.SYS in SYS:\LOGIN directory

BOOTCONF.SYS format is

0 x 52544C*         =   NE2000.SYS

0 x 52544C29*       =   RTL8029.SYS

(Node number)   RTL8029.SYS,NE2000.SYS renamed by NET$DOS.SYS file.

Note:

if have not BOOTCONF.SYS file or node number is not match in BOOTCONF.SYS

Node number , there will be boot default NET$DOS.SYS

7. Copy RBOOT.RPL, ETHER.RPL to the SYS:LOGIN directory.

8. Copy RPL.NLM to the SYS:SYSTEM directory.

Note: It depends on file server's version.

In the RPL developer's kit,

for Netware 3.11,  use .\RPL\NLM\311\RPL.NLM;

for Netware 3.12/4.x  use .\RPL\NLM\40\RPL.NLM;

9. Edit the AUTOEXEC.NCF file in the SYS:SYSTEM directory.

For example:

file server name FTEST

ipx internal net 831123

load NE2000 port=300 int=11 name=ftest_1 frame=ETHERNET_802.2

load NE2000 port=300 int=11 name=ftest_2 frame=ETHERNET_II

load NE2000 port=300 int=11 name=ftest_3 frame=ETHERNET_802.3

load NE2000 port=300 int=11 name=ftest_4 frame=ETHERNET_Snap

bind ipx to ftest_1 net=21

bind ipx to ftest_2 net=22

bind ipx to ftest_3 net=23

bind ipx to ftest_4 net=24

; ---------- for RPL file server -----------------

load rpl

bind rpl to ftest_1

; ---------- for RPL file server -----------------

mount all

load pburst

set NCP PACKET SIGNATURE OPTION = 1

set immediate purge of deleted files=on

load remote FRTSA

load rspx

load monitor

10. Shutdown the file server, and reboot it.

11. Configure the RTL8029 PCI Card to "Boot ROM Enable".

11. Now, when the file server is running, the workstation equipped with Boot

ROM on the RTL8029 card will be attached to the file server without any

hard disk or floppy.

C. How to install the Microsoft Windows NTAS 3.5 , 3.51 RPL server

1.Installing the Remoteboot Service on the Server.

(a).Choose Control Panel , Network , Add Software , Remoteboot Service

, put NTAS CD or disk they will be installed Remoteboot Service.

Note:

To install RPL server before , you must be had installed DLC and

NetBEUI protocol in your NTAS.

2.Sharing RPLFILES files and to install MS-DOS Files for Remoteboot

workstation.

(a).Sharing \\systemroot\rpl\rpl\rplfiles to V:\\RPLFILES

;use File manager sharing

(b).Copy all the MS-DOS 6.22 ver files to

----> \\systemroot\RPL\RPLFILES\BINFILES\DOS622

copy c:\dos\*.* V:\BINFILES\DOS622

attrib -s -h c:\io.sys

attrib -s -h c:\msdos.sys

copy c:\io.sys V:\BINFILES\DOS622

copy c:\msdos.sys V:\BINFILES\DOS622

attrib +s +h c:\io.sys

attrib +s +h c:\msdos.sys

3.Creating Remoteboot configuration for new adapter

(a).copy the MS-DOS device driver (NDIS2) for the RTL8029 PCI adapter to the

\\systemroot\RPL\BBLOCK\NDIS directory  (PCIND.DOS of RTL8029 NDIS2 driver)

(b).Create this directory \\systemroot\RPL\BBLOCK\NETBEUI\RTL8029 and

copy RTL8029.CNF , protocol.ini files

(c).use RPLCMD utility to add a bblock record for new adapter.

(d).In MS-DOS Prompt type <1>. NET START RemoteBoot

<2>. RPLCMD

(e).Screen will display under line option and you following these do it.

Adapter Boot Config Profile Service Vendor Wksta [Quite] V

Add Del Enum: A

VendorName=525429

VendorComment=Realtek for RTL8029

Adapter Boot Config Profile Service Vendor Wksta [Quite] B

Add Del Enum:A

BootName=DOSY

VendorName=525429

BbcFile=BBLOCK\NETBEUI\RTL8029\RTL8029.CNF

BootCommet=Name of this adapter

WindowsSize=0

Adapter Boot Config Profile Service Vendor Wksta [Quite] C

Add Del Enum:A

ConfigName=DOS622X

BootName=DOSY

DirName=DOS

DirName2=DOS622

FitShared=FITS\DOS622.FIT

FitPersonal=FITS\DOS622P.FIT

ConfigComment=DOS 6.22 <Adapter name>

DirName3=

DirName4=

(f).Shutdown Windows NTAS server, and reboot it.

4.Login Administrator to NTAS server.

5.Starting Remoteboot service and Remoteboot manager

(a).In MS-DOS Prompt type " NET START Remoteboot "

(b).In Network administrator , Remoteboot Manager

1.Choose Remoteboot , New Profile , created a RTL8029 Profile Name

2.Add new workstation for RTL8029 adapter and choose RTL8029 Profile

file.

6.To config RTL8029 card "Node ID (525429******)" and Running Boot ROM of

RTL8029 Card .

Note:

If the new workstation Node ID is not match for your workstation ,

you can use "Convert Adapters" in Remoteboot option.

D. How to install the Microsoft LAN Manager 2.1 Remoteboot server

1.Installing Microsoft OS/2 in the HD.

2.Installing Microsoft LAN Manager 2.1 in OS/2.

Note:

Network Driver protocol must have Netbeui , Remoteboot protocol .

3.Installing RPL server in LAN Manager 2.1

(a).IN OS2 Full screen mode type RPLSETP  and choose MS-DOS workstation.

Copy files from A:\

Install LAN MANAGER for MS-DOS Remoteboot workstation? Y

Install LAN MANAGER for MS OS/2 Remoteboot workstation? N

Replicate Remoteboot files ? N

Is the above information correct (Yes , No ,eXit ) ? Y

4.Installing MS-DOS Files for Remoteboot workstation.

(a).Copy all the MS-DOS files to the

------> \\systemroot\RPL\RPLFILES\BINFILES\DOS500

attrib -s -h c:\io.sys

attrib -s -h c:\msdos.sys

copy c:\io.sys \\systemroot\RPL\RPLFILES\BINFILES\DOS500

copy c:\msdos.sys \\systemroot\RPL\RPLFILES\BINFILES\DOS500

attrib +s +h c:\io.sys

attrib +s +h c:\msdos.sys

5.Creating remoteboot configuration for new adapter

(a).copy the MS-DOS device driver for the network adapter to the

\\systemroot\RPL\BBLOCK\NDIS directory (PCIND.DOS of RTL8029 NDIS2 driver)

(b).Create the directory \\systemroot\RPL\BBLOCK\NETBEUI\RTL8029 and

copy RTL8029.CNF , protocol.ini files

(c).add these contents to RPL.MAP file :

;boot bblock records:

yyyyyyyyyyyy BBLOCK\NETBEUI\RTL8029.CNF 2 6 F ~ DOS Realtek ~RTL8029

52544C29~ ,,,, ~ RDOSY

;default workstation records:[enable only one in each address range]

52544C29???? ?????? D FITS\DEFAULT [RPL_SERVER_NAME] S ~~~ ,,, ~RDOSY~

DEFAULT *~Realtek RTL8029 ~* ~~~

;profile and workstation records added by rplmgr:

(d).In OS/2 Full Screen type "NET" and logon supervisor to Network

1.In OS/2 Full Screen type "RPLINST" !!! wait for minutes ....

2.Choose View , Available resource , RTLFILES , Use resource ,

Local device [ D:]

6.Starting Remoteboot service and Remoteboot manager

(a).In MS-DOS Prompt type " NET START Remoteboot "

(b).In OS/2 Full Screen type " RPLMGR " (RPL MANAGER utility)

1 Choose Remoteboot , New Profile , created a RTL8029 Profile Name

2 Add new workstation for RTL8029 adapter and choose RTL8029 Profile

file.

7.To config RTL8029 card "node id (52544C29*****)" and Running Boot ROM of

RTL8029 Card .

Note:

If the new workstation Node ID is not match for your workstation ,

you can use "Convert Adapters" in Remoteboot option.

E. How to install the IBM OS/2 LAN SERVER 4.0 Remoteboot server.

1.Installing IBM OS/2 in the HD.

2.Installing IBM LAN Server 4.0 in OS/2.

Note:

LAN Adapter and protocol support must have IBM IEEE 802.2 , NETBIOS

protocol.

3.Installing RPL server in LAN Server 4.0

(a).Choose LAN Services , OS/2 LAN Services installation/Configuration

Advanced , install or configure this workstation , Domain control

install or remove a component , add "DOS Remote IPL Service" , config

DOS Remote IPL Service , add MS-DOS 5.0 OK

4.Creating Remoteboot configuration for new adapter

(a).copy the MS-DOS device driver for the network adapter to the

\\IBMLAN\RPL\DOS directory (PCIND.DOS) and RTL8029.CNF

(b).Create this directory \\IBMLAN\RPL\DOS\RTL8029 and copy protocol.ini

file.

(c).add these contents to RPL.MAP file :

;server records:

yyyyyyyyyyyy RTL8029.CNF 3 10 A IBMLAN$ DOS ~ Realtek ~RTL8029

~PCI~Adapter~ ~ ~,,,, R_RTL8029 ~ ~

; workstation records

52544C290001 RTL8029 ~ RT29IMG LANS4 CNSDS2 ~ ~ ~ ,,, Z R_RTL8029 ~ ~ ~

(d).logon supervisor to LAN Server 4.0

1.In OS/2 Full Screen type "GETRPL" !!! wait for  ....

2.Choose LAN Service , LAN Requester , Definition , IPL Images

STH5H500 , Actions , Create ,Image ID = RTL8029 .

3.Definition , Machine paramenters , NEW , Action , Create ,

Remote IPL workstation

Machine ID  . . . . . . . . . . . . . [ RTL8029]

Description . . . . . . . . . . . . . [ Realtek RTL8029 PCI Adapter]

Network adapter number  . . . . . . . [ 52544C290001               ]

Remote IPL server . . . . . . . . . . [ F4 ]

Server record identifier. . . . . . . [ F4 or R_RT8029 ]

5.Starting Remoteboot service and Remoteboot manager

(a).In MS-DOS Prompt type " NET START Remoteboot "

6.To config RTL8029 card "Node ID (52544C290001)" and Running Boot ROM of

RTL8029 Card.